2025. 11

제조데이터 표준 활용 가이드(취출로봇)

AAS표준(IEC 63278-1:2023)을 중심으로

목 차

I. AAS 개요	4

1. AAS 개념	4

2. AAS 구성요소	6

3. AAS 작성 요구사항	10

4. AAS 작성 유의사항	11

5. AAS 모델링	12

II. 장비 개요	15

1. 취출기 정의 및 종류	15

III. AAS 참조모델	22

1. AAS 참조모델 개요	22

2. AAS 참조모델 세부 구성	22

3. AAS 참조모델 근거자료	22

IV.AAS 참조모델 활용 방법	130

1. 참조모델 활용	130

2. 신규 AAS 작성 및 수정	130

3. AAS 모델(템플릿)을 실제 제품별 AAS 인스턴스로 전환	138

4. 분말 성형 공정 AAS 참조모델 활용 시나리오	142

표 목차

표 1 ISO/IEC 표준 제정 절차	5

표 2 AAS 모델링 절차	12

표 3 AAS 모델링에 사용한 서브 모델과 버전	13

표 4 취출기 작동 주기	16

표 5 참조모델의 Submodels 개요	24

그림 목차

그림 1 SCI 4.0의 주요 워킹 그룹 및 국제협력	4

그림 2 AASX Package Explorer로 나타낸 6축 로봇의 AAS	6

그림 3 AAS, submodel, submodel collection, property의 관계	7

그림 4 IRDI를 사용한 식별자	8

그림 5 URI, URL의 구조	8

그림 6 ecl@ss 홈페이지	9

그림 7 CDD 홈페이지	9

그림 8 데이터 수집 플랫폼에서 사용하는 URI 형식	12

그림 9 AAS 모델링 절차	13

그림 10 취출기 장비 예시	15

그림 11 ISO 10218-2	19

그림 13 eCl@ss Cartesian robot(27-38-01-03)	21

그림 14 취출로봇 참조모델 구성	22

그림 15 eCl@ss(27-38-01-03)	23

그림 16 AASX PackageExplorer 다운로드 경로	130

그림 17 AASX Package Explorer 파일 실행 초기화면	131

그림 18 활성화된 Edit 모드	131

그림 19 Edit 모드 활성화 단계	132

그림 20 AAS 생성	132

그림 21 AAS 정보	133

그림 22 IDTA SubmodelTemplate 다운로드	133

그림 23 IDTA SubmodelTemplate 파일 열기	134

그림 24 Template 이용 Submodel 생성	135

그림 25 동일한 방법으로 추가한 3종의 Submodel	135

그림 26 Submodel 생성	136

그림 27 SubmodelCollection 생성	136

그림 28 SubmodelCollection 정보	137

그림 29 Property 생성	137

그림 30 AssetInformation의 kind 변경	138

그림 31 AssetInformation의 globalAssetId 입력	138

그림 32 AssetInformation의 assetType 입력	139

그림 33 Submodel 인스턴스	139

그림 34 Property 인스턴스	140

그림 35 AAS 인스턴스 변환 전 화면	140

그림 36 AAS 인스턴스 변환 후 화면	141

그림 38 분말 성형 공정 AAS 참조모델 활용 시나리오	142

그림 39 자산 식별 및 디지털 트윈 온보딩	142

그림 40 공정 실시간 모니터링 및 이상 탐지	143

그림 41 소결 공정 에너지 효율 최적화	144

그림 42 생산 이력 기반 품질 역추적	144

그림 43 데이터 기반 예지보전	145

그림 44 장비 간 협동 제어 및 자율 구성	146

그림 45 원재료 기반의 지능형 공정 제어 및 품질 추적	147

그림 46 예지 보전 및 에너지 효율 관리	147

그림 47 장비 라이프사이클 및 구성 변경 관리	148

I. AAS 개요

1. AAS 개념

가. Industrie 4.0

○ Industrie 4.0은 유연하고 효율적인 생산을 위해 정보통신기술(ICT)을 활용하여 산업용 장비와 공정을 네트워크화하는 ‘산업 부문의 디지털 전환’ 전략을 말하며, 이러한 Industrie 4.0의 목표는 사이버 물리시스템(CPS: Cyber Physical System) 기술 및 제품 시장에서의 우위를 확보하는 것이다.

○ Industrie 4.0을 구현하기 위해서는 표준화가 핵심적으로 중요하다. Industrie 4.0은 도메인, 계층 경계 및 생애 주기 단계에 걸쳐 전례 없는 시스템 통합을 요구한다. 독일은 RAMI 4.0 (Reference Architecture Model Industrie 4.0)을 통해 국제 표준화 과정에서 모델을 제공하고 있으며, 이를 통해 재빠른 표준화를 바탕으로 국제적 협력을 촉진하는 역할을 수행하고 있다.

○ 이러한 Industrie 4.0을 실현하기 위한 핵심 요소 기술로 AAS가 주목받고 있다.

○ 관련 기구
- 16년 하노버 박람회에서 독일 산업계 표준화 기관인 Bitkom(Bundesverband Informationswirtschaft, Telekommunikation und neue Medien e. V.), DIN(DeutscheInstitut für Normung e. V) 등이 “SCI(Standardization Council Induestire) 4.0” 이니셔티브 설립을 발표하였다. 해당 표준화위원회는 Industry 4.0 관련 표준화를 시작하고 국내 및 국제적으로 표준을 조정하는 것을 목표로 한다.

그림 1 SCI 4.0의 주요 워킹 그룹 및 국제협력

- ISO는 1947년에 설립된 국제표준화기구로, 다양한 산업 및 기술 분야에서 전 세계적으로 일관성 있고 호환 가능한 표준을 개발하는 것을 목표로 하는 비영리 기구이다. ISO는 ISO정관(Statute) 제2조에 명기된 바와 같이 상품 및 서비스의 국제적 교환을 촉진하고, 지적, 과학적, 기술적, 경제적 활동 분야에서의 협력 증진을 위하여 세계의 표준화 및 관련 활동의 발전을 촉진시키는데 있다. 이러한 목적 달성을 위하여, ISO는 표준 및 관련 활동의 세계적인 조화를 촉진시키기 위한 조치를 취하며 국제표준을 개발, 발간하며 해당 표준이 세계적으로 사용되도록 조치를 취하고, 표준화 사업에 관한 연구를 통해 타 국제기구와 협력한다.

- IEC(International Electrotechnical Commission)는 1906년에 설립된 비영리 국제 표준화 기구로, 전기 및 전자 기술 분야에서 표준에 대한 준수 확인 등과 같은 표준화에 대한 제반 현안 및 관련 사항에 대한 국제간 협력을 촉진하여 국제간의 이해를 증진시키는 것을 목적으로 한다. IEC의 표준화 작업은 ISO와 공동으로 제정한 ISO/IEC Directive에 따라 이루어지므로 아래 표와 같이 ISO의 표준화 작업 절차와 대동소이하다. 다만 질의단계 문서를 ISO에서는 DIS라 부르는 반면 IEC는 CDV라 부르는 점이 다르다. 기타 특이 사항은 Directive의 IEC 부속서에 설명되어 있다.

표 1 ISO/IEC 표준 제정 절차

| 프로젝트 단계 | 관 련 문 서 |  |
| --- | --- | --- |
|  | 명 칭 | 약 어 |
| 0 예비단계 | 예비 작업 항목 | PWI |
| 1 제안단계 | 신규 작업 항목 제안 | NP |
| 2 준비단계 | 작업 초안 | WD |
| 3 위원회단계 | 위원회안 | CD |
| 4 질의단계 | 질의안/투표용 위원회안 | DIS/CDV |
| 5 승인단계 | 최종 국제표준안 | FDIS |
| 6 출판단계 | 국제표준 | IS |

나. AAS 개념

○ Asset Administration Shell(AAS)은 물리적 자산을 디지털로 표현하여 정보와 동작을 표준화하고 통합 관리하는 구조이다. 이 개념은 자산의 전체 라이프사이클 동안 설계, 제조, 유지보수, 폐기에 이르는 모든 데이터를 디지털화하여 관리함으로써 상호운용성과 데이터 교환을 지원하는 데 중점을 둔다. 특히, AAS는 다양한 자산의 상태, 특성 및 동작을 디지털 트윈 형태로 구현하여, 실시간 모니터링 및 최적화를 가능하게 하고, 정보의 일관성과 신뢰성을 높이며, 다른 시스템과의 통합을 용이하게 한다. 이를 통해 제조 및 운영의 효율성을 향상시키고, Industrie 4.0의 핵심 기술로 자리 잡고 있다.

○ IEC 63278-1에서는 자산의 표준화된 디지털 표현 구조인 AAS를 정의한다. IEC 63278-1은 AAS의 목적을 ‘두 개 이상의 소프트웨어 애플리케이션이 정보를 교환하고, 교환된 정보를 신뢰할 수 있고 안전한 방식으로 상호 활용할 수 있도록 하는 것’이라고 명시하고 있다.

○ AAS를 보다 원활하게 제작하기 위해 AASX Package Explorer라는 오픈소스 소프트웨어 툴이 AAS 표준과 함께 개발이 진행되고 있다. 이후 버전에 따라 형식 및 기능이 달라질 수 있다. 아래 그림은 AASX Package Explorer를 활용하여 AAS를 표현한 하나의 사례이다.

그림 2 AASX Package Explorer로 나타낸 6축 로봇의 AAS

2. AAS 구성요소

가. Submodel

○ Submodel은 물리적 자산이 가지는 다양한 특성을 표현하는 데이터 모음이다. 모든 AAS는 하나 이상의 submodel로 구성되며, submodel을 구분하기 위한 지정된 기준은 존재하지 않는다.

나. Property

○ Property는 자산의 고유한 특성을 기술하는 요소이다. 모든 Property는 고유 식별자를 가지며, 같은 의미로 일관되게 해석될 수 있어야 한다. Property의 고유성 보장을 위해 적어도 하나의 고유한 명칭과 ID를 부여하며, 도메인 내에서는 동일한 의미로만 정의되어야 한다.

다. Collection

○ Collection은 유사한 특징을 가지는 여러 Submodel Element들을 그룹화한 모음이다. 특정 submodel 내에서 유사한 특성을 공유하는 property들이 모여 하나의 collection을 구성하며, submodel의 관리와 데이터 구조화에 유용하게 사용된다.

○ AAS와 submodel, collection, property의 관계는 아래 그림과 같이 나타낼 수 있다.

그림 3 AAS, submodel, submodel collection, property의 관계

라. AAS Identifier

○ 모든 AAS는 다른 AAS와 구분되어야 하며, 상호 데이터 교환 및 인터페이스가 가능해야 한다.

○ IRDI
- IRDI(International Registration Data Identifier)는 국제적으로 공인된 데이터 사전(CDD, eCl@ss 등)에서 관리하는 국제 등록 식별자로, 각 기관에 의해 부여된 식별자이기 때문에 전 세계적으로 고유한 식별자임이 보장된다. 아래의 그림은 ISO 29002-5에 따른 IRDI의 구조를 나타낸다.

그림 4 IRDI를 사용한 식별자

○ URI
- URI(Uniform Resource Identifier)는 계층적으로 경로를 지정할 수 있는 구조로, 공인된 기관에 의해 식별자가 부여되지 않더라도 전 세계적으로 고유한 식별자를 정의할 수 있다. IRDI로 정의할 수 없는 property, submodel, AAS 등의 식별자에는 일반적으로 URI를 사용한다. 보통 인터넷에서 많이 사용되고 있는 식별자인 URL(Uniform Resource Locators)도 URI의 일종이다.

그림 5 URI, URL의 구조

마. Semantic

○ Semantic은 데이터의 의미와 해석을 보장하는 요소로, 사람이든 기계든 동일한 의미로 이해할 수 있도록 정의된다. 하나의 property는 도메인 내에서 하나의 의미로만 정의되며, eCl@ss, CDD 등 데이터 사전에 등록된 고유한 의미를 통해 일관된 semantic을 갖는다.

○ eCl@ss
- eCl@ss는 전 세계 다양한 산업의 제품, 서비스, 소재, 시스템을 분류하고 기술하는 데 사용되는 ISO/IEC 규격 제품 데이터 표준으로, 데이터에 대한 메타데이터와 사람과 기계 모두 이해 할 수 있는 통일된 언어를 제공한다.

그림 6 ecl@ss 홈페이지

○ CDD (Common Data Dictionary)
- IEC 61360 공통 데이터 사전(CDD)은 전기 전자 산업 전반에서 사용되는 공통의 용어 및 개념을 저장하는 저장소로, IEC 국제표준이나 업계 표준 등을 기반으로 일반적으로 사용하는 용어 및 개념을 정의하고 있다.

그림 7 CDD 홈페이지

바. Concept Description

○ Concept Description이란 개념 사전을 구성하는 개별 항목들을 의미하며, AAS에서 사용되는 하나의 property에 연결된 property의 semantic을 의미한다. 개념사전은 AAS의 모든 property들이 가지는 semantic의 집합으로, 마치 사전처럼 모든 property의 정의와 형식, 단위 등을 표현한다. AAS의 모든 property는 Concept Description을 통해 의미가 정의되고. CDD, eCl@ss와 같은 데이터베이스에서 제공하는 표준 데이터 형식에 기반하여, property의 의미와 정의, 단위 등을 설명하는 사전 역할을 한다.

3. AAS 작성 요구사항

가. AAS의 필수 구성요소

○ 하나의 AAS는 하나 이상의 submodel로 구성되어야 한다. AAS를 구성하는 submodel에 대한 리스트를 제한하거나 필수 submodel을 규정하지는 않고 있지만, 일반적으로 사용되는 submodel에 대한 가이드라인을 기준으로 몇 가지 주요 submodel을 요약할 수 있다. 주요 submodel은 아래와 같다.

나. 주요 submodel

○ Identification submodel
- Identification submodel은 AAS의 기본이 되는 submodel로, AAS 표준에서 필수로 지정되지는 않았으나 모든 AAS가 반드시 가져야 하는 submodel이다. 자산을 식별하기 위한 고유 번호, 자산에 대한 설명, 판매 정보, 제조 정보 등 다양한 정보를 포함하며, 포함되는 정보에 대한 제한은 없다.

○ Digital Nameplate submodel
- Digital Nameplate submodel은 자산의 물리적인 명판(Nameplate)을 디지털 형태로 표준화하여 표현하는 submodel이다. submodel에는 제조사명, 제품 명칭, 시리얼 번호, 제조 연도, 버전 정보 등 자산의 핵심 식별 정보와 사양 정보가 포함된다. 이 submodel은 IEC 61406 표준을 따르며, 자산의 전 생애주기에 걸쳐 명확한 정보를 제공하여 이력 관리 등을 용이하게 한다.

○ Technical data submodel
- Technical data submodel은 자산의 기술적인 특징과 매개변수를 의미하는 기술사양 정보를 포함한다. 일반적으로 해당 submodel에 포함된 데이터들은 제조 현장으로부터 실시간으로 수집되지 않는 데이터들이다.

○ Operational data submodel
- Operational data submodel은 제조 현장에서 실시간으로 수집되는 동작 정보를 포함하며, 실제 공정 또는 제조 설비에서 수집되는 데이터를 의미한다. 해당 submodel의 명칭이 반드시 ‘Operational Data’일 필요는 없다. 그러나 일반적인 정보들의 경우, ‘Operational Data’라는 명칭을 사용하여 실시간 데이터를 다룬다는 점을 명확히 하는 것이 권장된다.

○ HandoverDocumentation submodel
- HandoverDocumentation submodel은 자산과 함께 제공되는 사용 설명서, 설치 매뉴얼, 인증서, 절차서 등 문서 정보를 구조화하여 제공하는 서브모델이다. VDI 2770 표준 기반으로 문서 식별, 버전, 분류, 파일 경로 등을 포함하여, 장비 인수(handover) 및 유지보수 시 필요한 공식 문서를 AAS 형태로 전달하기 위해 사용된다.

○ HierarchicalStructresEnablingBoM submodel
- HierarchicalStructures(Enabling BoM) submodel은 자산의 구성요소(부품·모듈) 관계를 구조적으로 표현하는 Bill of Materials(BoM) 역할을 한다. 상위 장비에서 하위 컴포넌트로 이어지는 계층 구조(hierarchy)를 AAS로 기술하며, 장비 구성의 추적성, 관리, 유지보수 계획 수립 등을 지원한다.

○ GripperManagement submodel
- GripperManagement submodel은 취출 로봇의 그리퍼(Gripper) 사양 및 유지보수 이력 데이터를 관리하기 위한 장비 특화 서브모델로, 해당 서브모델의 프로퍼티들은 로봇 그리퍼 관련 일반 산업 사양 및 유지보수 관리 기준에 근거하여 모델링하였다.

○ PredictiveMaintenance submodel
- PredictiveMaintenance submodel은 생산 시스템 및 관련 소프트웨어의 예지 보전(Predictive Maintenance) 구현에 필요한 정보를 정의하는 서브모델이다. OPC Foundation 표준 기반의 잔존 수명(RUL) 예측값과 예측에 사용된 AI 모델의 유형 및 상세 사양을 구조화하여 관리한다. 또한 예측의 유효성을 결정하는 경계 조건과 임계점 도달 전후의 단계별 알림 체계를 포함하여, 설비 고장 예방 및 정비 시점 최적화를 위한 디지털 기초 데이터를 제공한다.

○ AIModelNameplate submodel
- AIModelNameplate submodel은 자산에 적용된 AI 모델의 식별 정보와 기술적 사양을 표준화된 형태로 제공하는 서브모델이다. 모델의 버전, 저장 경로, 학습 방법(지도/비지도 학습 등)과 같은 기본 관리 정보뿐만 아니라, 입력 데이터의 유형(이미지, 오디오 등) 및 출력 차원의 의미를 포함한 모델의 입출력 구조를 상세히 기술한다. 또한 프레임워크, 프로그래밍 언어, 하이퍼파라미터 등 모델의 아키텍처 정보와 학습 결과(Plots)를 포함하여, AI 모델의 투명성을 확보하고 자산 내에서 AI 모델의 원활한 운용 및 유지보수를 지원하는 디지털 기초 데이터를 제공한다.

4. AAS 작성 유의사항

○ eCl@ss, CDD 등에서 적절한 Semantics가 정의되지 않은 경우
- AAS의 모든 property는 semantic이 필요하지만, 반드시 eCl@ss/CDD 등에 정의된 semantic을 사용해야 하는 것은 아니다. AAS는 고유한 식별자를 활용하여 모든 property의 의미가 유일하게 정의되고 구분되는 것을 요구한다. 따라서 eCl@ss/CDD 등에 적절한 semantic이 없을 경우 자체적으로 이를 정의하여 사용해야 한다.

- 자체적으로 semantic을 정의하는 경우 URI 형식의 식별자를 사용하여야 한다. URI를 사용할 경우에는 아래의 형식을 따라야 한다.

그림 8 데이터 수집 플랫폼에서 사용하는 URI 형식

○ eCl@ss/CDD에 정의된 Semantics의 단위나 데이터 타입이 다른 경우
- property의 semantic은 해당 semantic이 정의하고 있는 모든 사항을 그대로 적용하는 경우에만 의미가 있으므로, 데이터 타입, 단위 등이 다른 경우 역시 별도로 semantic을 정의하여야 한다.

5. AAS 모델링

○ AAS를 구현 및 활용하기 위해 일반적으로 아래 표와 그림과 같은 절차를 따른다.

표 2 AAS 모델링 절차

| 순서 | 단계 | 설명 |
| --- | --- | --- |
| 1 | 목표 설정 | 시스템 요구사항을 분석하고 활용 여부에 대해 파악한다 |
| 2 | 대상 탐색 | 모델링할 자산에 대해 충분히 이해한 상태에서 정보 수집한다 |
| 3 | Asset 설계 | 수집된 정보에 따라 Asset의 구조를 정의, 모델링한다 |
| 4 | AAS 설계 | 설계한 Asset을 기반으로 AAS를 설계한다. 여기서 설계한 AAS는 AAS를 구성하는 Asset, Submodel, Property 같은 요소들로 이뤄진다 |
| 5 | AAS 구현 | AAS 모델링 툴을 활용하여 실제 패키지 형태의 파일로 생성하며, AAS에 포함되는 모든 Submodel, Property와 같은 요소에 고유 ID (Semantics)가 부여되어야 한다 |
| 6 | 시스템 구현 및 검증 | 작성된 AAS 모델을 다양한 포맷으로 변환하고 시스템을 구현 및 검증한다 |

그림 9 AAS 모델링 절차

○ 본 가이던스에서 AAS 모델링을 위해 활용한 서브모델 표준 관련 버전 정보는 아래 표와 같다.

표 3 AAS 모델링에 사용한 서브 모델과 버전

| Submodel명 | 설명 | 내용 예시 | 비고 |
| --- | --- | --- | --- |
| Identification | 제품과 제조사에 대한 일반적인 정보 | Asset ID, 제조사 이름 | AasxPackageExplorer Submodel Plugin |
| DigitalNameplate for industrial equipment | 제품에 대한 분류 정보, 제조사 정보 등 제품에 대한 기본 정보. 제품의 인증 마킹 정보 등이 포함 | 시리얼번호, 제조 날짜, 인증마크 | IDTA 02006-3-0 Submodel_Digital Nameplate |
| TechnicalData | 제품의 기술적인 사양서, 데이터시트 정보 | 기술적 사양, 기능적 특성 | IDTA 02003-1-2 Submodel_TechnicalData |
| OperationalData | 자산의 운영 및 성능에 대한 실시간 데이터 및 상태 정보를 포함 | 실시간 상태 정보, 성능 데이터 |  |
| Hierarchical Structures enabling Bills of Material | 컴포넌트의 조합으로 구성된 제품의 계층적 구성을 포함한 정보 | 물리적 구성 요소, BoM | IDTA 02011-1-1 Submodel_HierarchicalStructures EnablingBoM |
| Handover Documentation | 제품의 브로셔, 설치/시운전/유지보수 매뉴얼 등 다양한 문서 정보 | 사용자 매뉴얼, 제품 카탈로그 | IDTA 02004-2-0 Submodel_HandoverDocumentation |
| Gripper Management | 그리퍼 관리 정보 | 사양, 유지보수 정보 |  |
| PredictiveMaintenance | 생산 시스템, 소프트웨어의 예지보전 정보 | 잔존 수명 예측값, AI 모델 유형 | IDTA 02048-1-0 Predictive Maintenance |
| AIModelNameplate | AI 모델의 식별 정보 및 기술적 사양 | 학습 방법, 데이터 유형, 아키텍쳐 정보 | IDTA 02060-1-0 Artificial Intelligence Model Namplate |
| AssetInterfaces Description | 자산의 통신 사양 정의 | 접속 지점, 보안 체계 구조, 데이터 전송 형식 | IDTA 02017-1-0 Asset Interfaces Description |

○ 본 가이던스에서 IRDI 부여를 위해 활용한 규격 버전 정보는 다음과 같다.

- Eclass v15 (2024.11 릴리즈)

- Eclass v14 (2023.11 릴리즈, v15에서 검색되지 않는 경우 v14 사용함)

  - 링크: ECLASS Basic & Advanced Content Search - ECLASS

- IEC CDD v2.0018.0002

  - IEC 61360-4 링크: IEC - Common Data Dictionary (CDD)

II. 장비 개요

1. 취출기 정의 및 종류

취출기는 분말사출성형공정에서 성형이 완료된 그린바디(Green Body)를 금형에서 안전하게 분리하여 꺼내는 장치이다. 분말사출성형품은 바인더에 의해 형상을 유지하는 상태로 기계적 강성이 약하여 파손되기 쉽기 때문에, 일반 플라스틱 사출성형보다 더욱 세심한 취출 과정이 필요하다.

그림 10 취출기 장비 예시

일반적으로 취출기는 X-Y-Z 직선운동을 통해 성형품을 취출하는 직교 로봇방식이 대부분이나, 성평품의 형상과 공정의 특수성 등으로 인해 다관절 로봇형식, 스카라 로봇 형식의 4축 로봇형 취출기도 사용된다.

가. 취출기 핵심 기능

바인더에 의한 결합으로 강도가 약한 성형품을 파손없이 분리하기 위해 힘을 제어하는 기술과 성형 사이클 단축을 통한 생산성 향상, 취출중 품질 유지 등을 위한 위치정확도와 자동 제어 기술이 필요하며 취출후 후공정 연계를 위한 이송공정 연계, 게이트 커팅 등의 후공정 연계 기능 등이 핵심 기능을 가지고 있다.

나. 취출기 주요 특징

분말사출성형 공정에서 사용되는 취출기는 강도가 약한 그린바디의 파손 취약성에 대응하기 위해 매우 정밀한 힘 제어가 가능하고, 성형품에 가해지는 압력을 최소화하면서도 균일하게 분산시킬 수 있다. 취출 과정은 로봇 그리퍼가 성형품을 안전하게 파지하여 다음 공정으로 이송한다. 이 과정은 성형기와 연동되어 자동으로 이루어지며, 각 단계마다 센서를 통해 위치와 상태를 모니터링한다. 자동화된 취출 시스템은 일정한 속도와 힘으로 반복 작업을 수행하여 품질의 균일성을 보장하고, 성형 사이클 시간을 단축하여 생산성을 크게 향상시킨다. 안전성 측면에서 취출기는 여러 보호 장치를 갖추고 있다. 비상정지 장치는 작업자가 위험 상황에서 즉시 모든 동작을 멈출 수 있게 하며, 센서 기반 충돌 감지 시스템은 예상치 못한 장애물이나 비정상적인 저항이 감지되면 자동으로 동작을 중지하고, 경보를 발생시켜 작업자에게 알린다. 유연성도 중요한 특징으로 분말성형 공장에서는 다양한 형상과 크기의 제품을 생산하는 경우가 많은데, 취출기는 제품이 바뀔 때마다 신속하게 재설정될 수 있어야 하며, 로봇 시스템의 경우 그리퍼를 교체하고 프로그램을 변경하는 것만으로 새로운 제품에 대응할 수 있다. 최신 시스템들은 복수의 공정을 동시에 처리할 수 있어, 취출 후 게이트 커팅, 검사, 적재 등의 후속 작업까지 통합적으로 자동화할 수 있다.

제어 시스템의 정밀성도 주요한 특징으로 서보 모터나 정밀 유압 시스템을 통해 밀리미터 단위의 위치 정확도를 구현하고, 힘 센서를 통해 그리퍼가 성형품에 가하는 압력을 실시간으로 모니터링하고 조절한다.

다. 취출기 작동 원리

취출기는 다음과 같은 주기로 작동한다.

표 4 취출기 작동 주기

| 순 서 | 설 명 |
| --- | --- |
| (1) 대기 및 신호 수신 | 성형이 진행되는 동안 취출기는 초기 위치에서 대기 |
| (2) 금형 진입 및 위치결정 | 금형 개방과 연동하여 로봇앞이 설정된 경로를 따라 금형 내부로 진입 |
| (3) 성형품 파지 | 그리퍼가 성형품에 도달하여 성형품을 안전하게 파지 |
| (4) 취출 및 이송 | 성형품을 안전하게 파지한 로봇은 금형에서 후퇴하여 설정된 경로를 따라 이동 |
| (5) 성형품 배출 및 복귀 | 목적지에 도달하면 그리퍼가 성형품을 배출한후 대기위치로 복귀 |

라. 취출기 주요 구성 요소

○ 로봇 본체

로봇 본체는 성형기에 고정되는 Base, 서보모터와 감속기로 정밀한 회전 및 직선운동을 구현하는 관절부, 작업 범위를 확장하는 경량 알루미늄 합금 재질의 암, 그리고 말단에 장착되어 실제로 성형품과 접촉하는 엔드 이펙터로 구성된다. 위치 센서가 내장되어 실시간으로 각도와 위치를 측정하며, 제어 시스템에 피드백을 제공하여 정확한 반복 작업을 수행한다.

○ 그리퍼/척

강도가 약한 분말성형품을 안전하게 파지하여 취출/이동하기 위해서 그리퍼는 성형품의 형상과 재질에 따라 선택되는 파지 장치로, 진공 흡착 패드는 평활한 표면을 음압으로 흡착하고, 기계식 그리퍼는 조가 물리적으로 파지하며, 자석식은 금속 분말 성형품을 자력으로 흡착한다. 맞춤형 지그는 복잡한 형상 제품 전용으로 설계됩니다. 모든 그리퍼는 힘 센서를 통해 파지력을 정밀 제어하여 분말사출 그린바디의 파손을 방지하면서 안전하게 파지한다.

○ 제어 시스템

로봇 컨트롤러는 고성능 프로세서로 복잡한 동작을 계산하고 실행하며, 직관적인 인터페이스를 통해 쉽게 프로그래밍할 수 있다. 위치, 압력, 온도, 비전 센서가 실시간으로 로봇 상태와 주변 환경을 감지하여 정확한 동작을 보장하며 리미트 스위치는 동작 범위를 제한하고 금형 개폐 상태를 확인하여 충돌을 방지한다.

마. 표준화 근거 자료

취출기는 다양한 산업 표준을 준수하여 설계 및 제작된다. 이는 장비의 안전성 확보, 데이터의 일관된 관리, 그리고 산업 내 상호운용성을 위한 기반이 된다. 본 장비와 관련된 주요 표준화 근거 자료는 다음과 같다.

○ ISO 10218-2 / KS B ISO 10218-2: 로봇 및 로봇 장치 - 산업용 로봇의 안전에 관한 요구사항

분말사출성형 취출 로봇 시스템의 설치, 통합 및 운영에 대한 안전 요구사항을 정의하는 핵심 표준이다. 이 표준은 로봇과 사출성형기 간의 안전한 통합, 협동 영역 설정, 안전 펜스 및 방호장치 설계, 비상정지 시스템, 인터록 장치 등 취출 로봇 시스템 전반의 안전 설계 및 위험성 평가 절차를 제시한다. 특히 분말사출성형의 경우 그린바디가 파손되기 쉬우므로 로봇의 안전한 작동과 작업자 보호가 동시에 요구되며, 본 표준은 이러한 요구사항을 충족하는 설계 근거를 제공한다.

○ ISO 9283 / KS B ISO 9283: 산업용 머니퓰레이팅 로봇 - 성능 항목 및 시험방법

취출 로봇의 위치 정확도(Pose accuracy), 반복 정밀도(Pose repeatability), 경로 정확도(Path accuracy), 속도 및 가속도 성능을 객관적으로 측정하고 평가하는 시험 방법을 규정한 표준이다. 분말사출성형 취출 작업에서는 그린바디를 정확한 위치에서 파지하고 후속 공정으로 정밀하게 이송해야 하므로, 로봇의 반복 위치 정확도가 매우 중요하다. 본 표준에서 정의한 ISO 테스트 큐브 방법론을 통해 취출 로봇의 성능을 검증하고, 제조사가 제시하는 성능 사양의 적합성을 확인할 수 있다.

○ IEC 60204-1 / KS C IEC 60204-1: 기계류의 안전성 - 기계의 전기장비 - 제1부: 일반 요구사항

취출 로봇 및 사출성형기를 포함한 모든 산업용 기계의 전기 장비에 대한 안전 요구사항을 규정하는 표준이다. 전기 회로 설계, 감전 방지, 접지, 배선, 비상정지 회로, 제어 전압, 전자기 적합성(EMC) 등 전기 안전의 모든 측면을 다룬다. 취출 로봇 시스템의 제어부, 센서, 액추에이터, 안전 회로 등은 모두 본 표준에 따라 설계되어야 하며, 특히 사출성형기와의 전기적 인터페이스 및 비상정지 기능의 안전 무결성을 확보하는 근거가 된다.

○ ISO 13849-1 / KS B ISO 13849-1: 기계안전 - 제어시스템의 안전 관련 부품

취출 로봇의 안전 관련 제어 시스템 설계에 대한 요구사항을 제시하는 표준으로, 성능 수준(Performance Level, PL) 개념을 통해 안전 기능의 신뢰성을 정량적으로 평가한다. 취출 시스템의 비상정지 회로, 안전 센서, 인터록 장치, 안전 PLC 등은 위험도 평가 결과에 따라 요구되는 PL(예: PLd 또는 PLe)을 만족하도록 설계되어야 한다. 본 표준은 안전 회로의 아키텍처, 고장 진단, 공통 원인 고장(CCF) 방지 등 안전 제어 시스템의 체계적 설계 방법론을 제공하여, 취출 로봇의 안전 무결성을 확보하는 기술적 근거가 된다.

바. 취출기 표준화 참고자료

○ ISO 10218-2 / KS B ISO 10218-2: 로봇 및 로봇 장치 - 산업용 로봇의 안전에 관한 요구사항

그림 11 ISO 10218-2

○ ISO 9283 / KS B ISO 9283: 산업용 머니퓰레이팅 로봇 - 성능 항목 및 시험방법

그림 12 ISO 9283

○ IEC 60204-1 / KS C IEC 60204-1: 기계류의 안전성 - 기계의 전기장비 - 제1부: 일반 요구사항

그림 13 IEC 60204-1

○ ISO 13849-1 / KS B ISO 13849-1: 기계안전 - 제어시스템의 안전 관련 부품

그림 14 ISO 13849-1

○ eCl@ss Standard – Search eCl@ss Contents

그림 13 eCl@ss Cartesian robot(27-38-01-03)

III. AAS 참조모델

1. AAS 참조모델 개요

○ AAS 참조모델이란 AAS에 익숙하지 않은 사용자들을 위해 AAS 작성을 돕기 위해 만들어진 AAS 모델이다.

○ AAS는 데이터를 발생시키는 개체를 대상으로 작성할 수 있으므로 하나의 AAS 하위에 여러 AAS가 존재할 수 있다.

○ 본 가이드라인에서는 앞서 소개한 장비인 취출로봇을 대상으로 구현한 AAS 모델을 참조 모델로 소개한다.

2. AAS 참조모델 세부 구성

○ 취출로봇 참조모델의 전체구조는 아래 <그림 24>과 같이 정의하였다. 필수적인 성격을 띠는 4가지 submodel 외에 4가지의 submodel을 추가하여 총 8가지의 submodel로 구성된다.

그림 14 취출로봇 참조모델 구성

3. AAS 참조모델 근거자료

○ AAS 참조모델의 경우 Property 등의 Semantic ID를 부여하기 위해 eCl@ss를 근거로 하여 모델링을 수행하였다.

그림 15 eCl@ss(27-38-01-03)

○ 각 submodel의 주요 내용은 아래 아래 표와 같이 구성된다.

표 5 참조모델의 Submodels 개요

| Submodel명 | 설명 | 내용 예시 | 비고 |
| --- | --- | --- | --- |
| Identification | 제품과 제조사에 대한 일반적인 정보 | Asset ID, 제조사 이름 | AasxPackageExplorer Submodel Plugin |
| DigitalNameplate for industrial equipment | 제품에 대한 분류 정보, 제조사 정보 등 제품에 대한 기본 정보. 제품의 인증 마킹 정보 등이 포함 | 시리얼번호, 제조 날짜, 인증마크 | IDTA 02006-3-0 Submodel_Digital Nameplate |
| TechnicalData | 제품의 기술적인 사양서, 데이터시트 정보 | 기술적 사양, 기능적 특성 | IDTA 02003-1-2 Submodel_TechnicalData |
| OperationalData | 자산의 운영 및 성능에 대한 실시간 데이터 및 상태 정보를 포함 | 실시간 상태 정보, 성능 데이터 |  |
| Hierarchical Structures enabling Bills of Material | 컴포넌트의 조합으로 구성된 제품의 계층적 구성을 포함한 정보 | 물리적 구성 요소, BoM | IDTA 02011-1-1 Submodel_HierarchicalStructures EnablingBoM |
| Handover Documentation | 제품의 브로셔, 설치/시운전/유지보수 매뉴얼 등 다양한 문서 정보 | 사용자 매뉴얼, 제품 카탈로그 | IDTA 02004-2-0 Submodel_HandoverDocumentation |
| Gripper Management | 그리퍼 관리 정보 | 사양, 유지보수 정보 |  |
| PredictiveMaintenance | 생산 시스템, 소프트웨어의 예지보전 정보 | 잔존 수명 예측값, AI 모델 유형 | IDTA 02048-1-0 Predictive Maintenance |
| AIModelNameplate | AI 모델의 식별 정보 및 기술적 사양 | 학습 방법, 데이터 유형, 아키텍쳐 정보 | IDTA 02060-1-0 Artificial Intelligence Model Namplate |
| AssetInterfaces Description | 자산의 통신 사양 정의 | 접속 지점, 보안 체계 구조, 데이터 전송 형식 | IDTA 02017-1-0 Asset Interfaces Description |

가. identification submodel의 하위 Property는 다음과 같이 구성된다.

| idShort: | Identification |  |  |
| --- | --- | --- | --- |
| Class: | Submodel |  |  |
| semanticId: | https://www.hsu-hh.de/aut/aas/identification |  |  |
| Parent: | Identification |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] ManufacturerName | 0173-1#02-AAO677#004 Manufacturer name Legally valid designation of the natural or judicial person which is directly responsible for the design, production, packaging and labeling of a product in respect to its being brought into circulation abc | [String] ExampleCompany Co., Ltd. | 0..1 |
| [Prop] ManufacturerId01 | 0173-1#02-AAY812#003 ManufacturerId01 Internationally unique identification number for the manufacturer of the device or the product and for the physical location - | [String] 8801234000017 | 0..1 |
| [Prop] ManufacturerIdProvider | 0173-1#02-AAP796#006 ManufacturerIdProvider DUNS-no., supplier number, or other number as identifier of an offeror or supplier of the identification - | [String] 123456789 | 0..1 |
| [Prop] ManufacturerTypId | 0173-1#02-AAO676#005 Manufacturer article number unique product identifier of the manufacturer - | [String] 04912345123457 | 0..1 |
| [Prop] ManufacturerTypName | 0173-1#02-AAW338#003 Manufacturer product designation Short description of the product (short text) - | [String] abcdefg | 0..1 |
| [Prop] ManufacturerTypDescription | 0173-1#02-AAU734#003 ManufacturerTypDescription Description of the product, it's technical features and implementation if needed - | [MLP] C-Frame crank press | 0..1 |
| [Prop] SupplierName | 0173-1#02-AAO735#005 SupplierName Name of supplier which provides the customer with a product or a service - | [String] ExampleCompany Co., Ltd. | 0..1 |
| [Prop] SupplierId | 0173-1#02-AAY813#003 SupplierId Internationally unique identification number for the supplier of the device or the product and for the physical location - | [String] 8805678000453 | 0..1 |
| [Prop] SupplierIdProvider | https://www.smart-factory.kr/ids/cd/SupplierIdProvider/1/0 SupplierIdProvider The unique identifier for the supplier. - | [String] 123456789 | 0..1 |
| [Prop] SupplierTypId | 0173-1#02-AAO736#006 SupplierTypId Unique product order identifier of the supplier - | [String] 04912345123457 | 0..1 |
| [Prop] SupplierTypName | 0173-1#02-AAM551#004 SupplierTypName Short description of the product - | [String] abcdefg | 0..1 |
| [Prop] SupplierTypDescription | 0173-1#02-AAU730#003 SupplierTypDescription Description of the product, it's technical features and implementation if needed - | [String] abcdefg | 0..1 |
| [Prop] TypClass | 0173-1#02-AAU731#003 TypClass Class of type or category - | [String] 00-00-00-00 | 0..1 |
| [Prop] ClassificationSystem | https://www.smart-factory.kr/ids/cd/ClassificationSystem/1/0 ClassificationSystem System used to categorize products or information. - | [String] ECLASS | 0..1 |
| [Prop] SecondaryKeyTyp | https://www.smart-factory.kr/ids/cd/SecondaryKeyTyp/1/0 SecondaryKeyTyp Type of key used for additional data categorization - | [String] CategoryCode | 0..1 |
| [File] TypThumbnail | https://www.smart-factory.kr/ids/cd/TypThumbnail/1/0 TypThumbnail Small preview image representing a type or category - | [File] thumbnail(ExampleCompany).png | 0..1 |
| [Prop] AssetId | https://www.smart-factory.kr/ids/cd/AssetId/1/0 AssetId A unique identifier for an asset - | [String] id | 0..1 |
| [Prop] InstanceId | https://www.smart-factory.kr/ids/cd/InstanceId/1/0 InstanceId A unique code for an instance or version of an asset - | [String] id | 0..1 |
| [Prop] ChargeId | 0173-1#02-AAQ196#003 Batch number Number assigned by the manufacturer of a material to identify the manufacturer's batch - | [String] BATCH-2099-01-0017 | 0..1 |
| [Prop] SecondaryKeyInstance | https://www.smart-factory.kr/ids/cd/SecondaryKeyInstance/1/0 SecondaryKeyInstance An additional unique identifier for an asset instance - | [String] 2099-00042 | 0..1 |
| [Prop] ManufacturingDate | 0173-1#02-AAR972#004 ManufacturingDate Date from which the production and / or development process is completed or from which a service is provided completely - | [String] 2099-01-01 | 0..1 |
| [Prop] DeviceRevision | https://www.smart-factory.kr/ids/cd/DeviceRevision/1/0 DeviceRevision An updated version of Device - | [String] Rev.02 | 0..1 |
| [Prop] SoftwareRevision | https://www.smart-factory.kr/ids/cd/SoftwareRevision/1/0 SoftwareRevision An updated version of Software - | [String] 1.4.2 | 0..1 |
| [Prop] HardwareRevision | https://www.smart-factory.kr/ids/cd/HardwareRevision/1/0 HardwareRevision A modified version or modification of Hardware for Problem correction - | [String] HW-Rev.03 | 0..1 |
| [File] QrCode | https://www.smart-factory.kr/ids/cd/QrCode/1/0 QrCode A two-dimensional barcode that stores information, easily scannable by smartphones and other devices - | [File] qrcode.png | 0..1 |
| [SMC] ContactInfo01 | https://www.smart-factory.kr/ids/cd/ContactInfo01/1/0 ContactInfo01 Basic contact information of a responsible person - | [-] | 0..1 |
| [File] CompanyLogo | https://www.smart-factory.kr/ids/cd/CompanyLogo/1/0 CompanyLogo A symbol made up of text and images that identifies a business - | [String] ExampleCompany.png | 0..1 |
| [Prop] URL | 0173-1#02-AAO694#003 URL Stated as link to a home page. The home page is the starting page or table of contents - | [String] ExampleCompany.com | 0..1 |

| idShort: | ContactInfo01 |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/ContactInfo01/1/0 |  |  |
| Parent: | ContactInfo01 |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Name | 0173-1#02-AAO677#004 Manufacturer name Legally valid designation of the natural or judicial person which is directly responsible for the design, production, packaging and labeling of a product in respect to its being brought into circulation - | [String] ExampleCompany Co., Ltd. | 0..1 |
| [Prop] Role | https://www.smart-factory.kr/ids/cd/Role/1/0 Role The specific part or function they have in the context - | [String] Headquarters | 0..1 |
| [SMC] PhysicalAddress | https://www.smart-factory.kr/ids/cd/PhysicalAddress/1/0 PhysicalAddress Basic physical address information. - | [-] 5 elements | 0..1 |
| [Prop] Email | https://www.smart-factory.kr/ids/cd/Email/1/0 Email The electronic mailing address - | [String] contact@ExampleCompany.com | 0..1 |
| [Prop] URL | 0173-1#02-AAO694#003 URL Stated as link to a home page. The home page is the starting page or table of contents - | [String] https://www.ExampleCompany.com | 0..1 |
| [Prop] Phone | https://www.smart-factory.kr/ids/cd/Phone/1/0 Phone The telephone contact number - | [String] +82-2-123-1234 | 0..1 |
| [Prop] Fax | https://www.smart-factory.kr/ids/cd/Fax/1/0 Fax The number used for sending/receiving documents electronically - | [String] +82-2-123-1235 | 0..1 |

| idShort: | PhysicalAddress |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/PhysicalAddress/1/0 |  |  |
| Parent: | PhysicalAddress |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] CountryCode | 0173-1#02-AAO730#003 CountryCode Agreed upon symbol for unambiguous identification of a country - | [String] KR | 0..1 |
| [Prop] Street | https://www.smart-factory.kr/ids/cd/Street/1/0 Street The street part of an address - | [String] 123 Example Street | 0..1 |
| [Prop] PostalCode | https://www.smart-factory.kr/ids/cd/PostalCode/1/0 PostalCode The code for mail delivery within their area - | [String] 12345 | 0..1 |
| [Prop] City | https://www.smart-factory.kr/ids/cd/City/1/0 City The city part of an address - | [String] Example City | 0..1 |
| [Prop] StateCounty | https://www.smart-factory.kr/ids/cd/Role/1/0 Role The specific part or function they have in the context - | [String] Example City | 0..1 |

나. DigitalNameplate submodel의 하위 SubmodelCollection, Property는 다음과 같이 구성된다.

| idShort: | DigitalNameplate |  |  |
| --- | --- | --- | --- |
| Class: | Submodel |  |  |
| semanticId: | https://admin-shell.io/idta/nameplate/3/0/Nameplate |  |  |
| Parent: | DigitalNameplate |  |  |
| Explanation: | Contains the nameplate information attached to the product@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] URIOfTheProduct | 0173-1#02-AAY811#001 URI of the product unique global identification of the product using an universal resource identifier (URI) - | [String] https://www.smart-factory.kr/ids/asset/MachinePress/1/0 | 1 |
| [MLP] ManufacturerName | 0112/2///61987#ABA565#009 ManufacturerName legally valid designation of the natural or judicial person which is directly responsible for the design, production, packaging and labeling of a product in respect to its being brought into circulation - | [String] Example Company@en | 1 |
| [MLP] ManufacturerProductDesignation | 0112/2///61987#ABA567#009 ManufacturerProductDesignation short description of the product (short text), third or lowest level of a 3 level manufacturer specific product hierarchy - | [String] abcdef@en | 1 |
| [SMC] AddressInformation | https://admin-shell.io/zvei/nameplate/1/0/ContactInformations/AddressInformation AddressInformation Note: this set of information is defined by SMT drop-in Note: this set of information is defined by SMT drop-in "Address Information" | [-] 4 elements | 1 |
| [MLP] ManufacturerProductRoot | 0112/2///61360_7#AAS011#001 ManufacturerProductRoot top level of a 3 level manufacturer specific product hierarchy - | [String] abcdef@en | 0..1 |
| [MLP] ManufacturerProductFamily | 0112/2///61987#ABP464#002 ManufacturerProductFamily second level of a 3 level manufacturer specific product hierarchy - | [String] abcdef@en | 0..1 |
| [Prop] ManufacturerProductType | 0112/2///61987#ABA300#008 ManufacturerProductType characteristic to differentiate between different products of a product family or special variants - | [String] abcdef | 0..1 |
| [Prop] OrderCodeOfManufacturer | 0112/2///61987#ABA950#008 OrderCodeOfManufacturer unique combination of numbers and letters issued by the manufacturer that is used to identify the device for ordering - | [String] 0000001 | 1 |
| [Prop] ProductArticleNumberOfManufacturer | 0112/2///61987#ABA581#007 ProductArticleNumberOfManufacturer unique product identifier of the manufacturer - | [String] MachinePress | 0..1 |
| [Prop] SerialNumber | 0112/2///61987#ABA951#009 SerialNumber unique combination of numbers and letters used to identify the device once it has been manufactured - | [String] 12345678 | 0..1 |
| [Prop] YearOfConstruction | 0112/2///61987#ABP000#002 YearOfConstruction year in which the manufacturing process is completed - | [String] 2099-01-01 | 0..1 |
| [Prop] DateOfManufacture | 0112/2///61987#ABB757#007 DateOfManufacture date when an item was manufactured - | [Date] 2099-01-01 | 0..1 |
| [Prop] HardwareVersion | 0112/2///61987#ABA926#008 HardwareVersion version of the hardware supplied with the device - | [String] 1.0 | 0..1 |
| [Prop] FirmwareVersion | 0112/2///61987#ABA302#006 FirmwareVersion version of the firmware supplied with the device - | [String] 1.0 | 0..1 |
| [Prop] SoftwareVersion | 0112/2///61987#ABA601#008 SoftwareVersion version of the software used by the device - | [String] 1.0 | 0..1 |
| [Prop] CountryOfOrigin | 0112/2///61987#ABP462#001 CountryOfOrigin country where the product was manufactured Note: Country codes defined accord. to DIN EN ISO 3166-1 alpha-2 codes | [String] Republic of Korea | 0..1 |
| [Prop] UniqueFacilityIdentifier | https://admin-shell.io/idta/nameplate/3/0/UniqueFacilityIdentifier UniqueFacilityIdentifier unique string of characters for the identification of locations or buildings involved in a product’s value chain or used by actors involved in a product’s value chain - | [String] FAC-2099-001 | 0..1 |
| [File] CompanyLogo | 0112/2///61987#ABP463#001 CompanyLogo a graphic mark used to represent a company, an organisation or a product - | [File] /aasx/Logo.png | 0..1 |
| [SML] Markings | 0173-1#01-AGZ673#001 Markings Collection of product markings Note: CE marking is declared as mandatory according to EU Blue Guide | [-] 3 elements | 0..1 |
| [SMC] AssetSpecificProperties | 0173-1#01-AGZ672#001 Asset specific Properties Group of properties that are listed on the asset's nameplate and are grouped based on guidelines - | [-] 0 elements | 0..1 |

| idShort: | AddressInformation |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/zvei/nameplate/1/0/ContactInformations/AddressInformation |  |  |
| Parent: | AddressInformation |  |  |
| Explanation: | Note: this set of information is defined by SMT drop-in "Address Information"@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [MLP] Street | 0173-1#02-AAO128#002 street street name and house number Note: see also [IRDI] 0112/2///61987#ABA286#001 street. Mandatory property according to EU Machine Directive 2006/42/EC | [String] 123 Example Street@en | 1 |
| [MLP] Zipcode | 0173-1#02-AAO129#002 zip code ZIP code of address Note: see also [IRDI] 0112/2///61987#ABA281#001 ZIP/Postal code. Mandatory property according to EU Machine Directive 2006/42/EC. Recommendation: property declaration as MLP is required by its semantic definition. As the property value is language independent, users are recommended to provide maximal 1 string in any language of the user’s choice. | [String] 12345@en | 1 |
| [MLP] CityTown | 0173-1#02-AAO132#002 city/town town or city Note: see also [IRDI] 0112/2///61987#ABA129#001 city/town. Mandatory property according to EU Machine Directive 2006/42/EC. | [String] Example City@en | 1 |
| [MLP] NationalCode | 0173-1#02-AAO134#002 national code code of a country Note: see also [IRDI] 0112/2///61360_4#ADA005#001 country code. country codes defined accord. to DIN EN ISO 3166-1 alpha-2 codes. Mandatory property according to EU Machine Directive 2006/42/EC. Recommendation: property declaration as MLP is required by its semantic definition. As the property value is language independent, users are recommended to provide maximal 1 string in any language of the user’s choice. | [String] KR@en | 1 |

| idShort: | Markings |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | 0173-1#01-AGZ673#001 |  |  |
| Parent: | Markings |  |  |
| Explanation: | Note: CE marking is declared as mandatory according to EU Blue Guide@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] - | 0173-1#01-AGZ673#001 Markings Collection of product markings Note: see also [IRDI] 0112/2///61987#ABH515#003 Certificate or approval Note: CE marking is declared as mandatory according to the Blue Guide of the EU-Commission | [-] 6 elements | - |
| [SMC] - | 0173-1#01-AGZ673#001 Markings Collection of product markings Note: see also [IRDI] 0112/2///61987#ABH515#003 Certificate or approval Note: CE marking is declared as mandatory according to the Blue Guide of the EU-Commission | [-] 6 elements | - |
| [SMC] - | 0173-1#01-AGZ673#001 Markings Collection of product markings Note: see also [IRDI] 0112/2///61987#ABH515#003 Certificate or approval Note: CE marking is declared as mandatory according to the Blue Guide of the EU-Commission | [-] 6 elements | - |

| idShort: | - |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | 0173-1#01-AGZ673#001 |  |  |
| Parent: | - |  |  |
| Explanation: | Note: see also [IRDI] 0112/2///61987#ABH515#003 Certificate or approval Note: CE marking is declared as mandatory according to the Blue Guide of the EU-Commission @en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] MarkingName | https://admin-shell.io/zvei/nameplate/2/0/Nameplate/Markings/Marking/MarkingName common name of the marking Note: see also [IRDI] 0173-1#02-BAB392#015 certificate/approval valueId with ECLASS enumeration IRDI is preferable, e.g. [IRDI] 0173-1#07-DAA603#004 for CE. If no IRDI available, string value can also be accepted. Note: CE marking is declared as mandatory according to Blue Guide of the EU-Commission | [String] 0173-1#07-DAA603#004 | 1 |
| [Prop] DesignationOfCertificateOrApproval | 0112/2///61987#ABH783#001 identifier of certificate, approval, declaration or license alphanumeric character sequence identifying a certificate or approval Note: Approval identifier, reference to the certificate number, to be entered without spaces | [String] Declaration of Conformity | 0..1 |
| [Prop] IssueDate | https://admin-shell.io/zvei/nameplate/2/0/Nameplate/Markings/Marking/IssueDate Date, at which the specified certificate is issued Note: format by lexical representation: CCYY-MM-DD Note: to be specified to the day | [String] 2099-01-01 | 0..1 |
| [Prop] ExpiryDate | https://admin-shell.io/zvei/nameplate/2/0/Nameplate/Markings/Marking/ExpiryDate Date, at which the specified certificate expires Note: see also ([IRDI] 0173-1#02-AAO997#001 Validity date Note: format by lexical representation: CCYY-MM-DD Note: to be specified to the day | [String] 2099-01-01 | 0..1 |
| [File] MarkingFile | https://admin-shell.io/zvei/nameplate/2/0/Nameplate/Markings/Marking/MarkingFile conformity symbol of the marking - | [File] /aasx/ce_marking.png | 1 |
| [Prop] MarkingAdditionalText | https://admin-shell.io/zvei/nameplate/2/0/Nameplate/Markings/Marking/MarkingAdditionalText where applicable, additional information on the marking in plain text, e.g. the ID-number of the notified body involved in the conformity process Note: see also [IRDI] 0173-1#02-AAM954#002 details of other certificate | [String] Declaration of Conformity | 0..* |

| idShort: | - |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | 0173-1#01-AGZ673#001 |  |  |
| Parent: | - |  |  |
| Explanation: | Note: see also [IRDI] 0112/2///61987#ABH515#003 Certificate or approval Note: CE marking is declared as mandatory according to the Blue Guide of the EU-Commission @en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] MarkingName | https://admin-shell.io/zvei/nameplate/2/0/Nameplate/Markings/Marking/MarkingName common name of the marking Note: see also [IRDI] 0173-1#02-BAB392#015 certificate/approval valueId with ECLASS enumeration IRDI is preferable, e.g. [IRDI] 0173-1#07-DAA603#004 for CE. If no IRDI available, string value can also be accepted. Note: CE marking is declared as mandatory according to Blue Guide of the EU-Commission | [String] 0173-1#07-DAA603#004 | 1 |
| [Prop] DesignationOfCertificateOrApproval | 0112/2///61987#ABH783#001 identifier of certificate, approval, declaration or license alphanumeric character sequence identifying a certificate or approval Note: Approval identifier, reference to the certificate number, to be entered without spaces | [String] Declaration of Conformity | 0..1 |
| [Prop] IssueDate | https://admin-shell.io/zvei/nameplate/2/0/Nameplate/Markings/Marking/IssueDate Date, at which the specified certificate is issued Note: format by lexical representation: CCYY-MM-DD Note: to be specified to the day | [String] 2099-01-01 | 0..1 |
| [Prop] ExpiryDate | https://admin-shell.io/zvei/nameplate/2/0/Nameplate/Markings/Marking/ExpiryDate Date, at which the specified certificate expires Note: see also ([IRDI] 0173-1#02-AAO997#001 Validity date Note: format by lexical representation: CCYY-MM-DD Note: to be specified to the day | [String] 2099-01-01 | 0..1 |
| [File] MarkingFile | https://admin-shell.io/zvei/nameplate/2/0/Nameplate/Markings/Marking/MarkingFile conformity symbol of the marking - | [File] /aasx/ce_marking.png | 1 |
| [Prop] MarkingAdditionalText | https://admin-shell.io/zvei/nameplate/2/0/Nameplate/Markings/Marking/MarkingAdditionalText where applicable, additional information on the marking in plain text, e.g. the ID-number of the notified body involved in the conformity process Note: see also [IRDI] 0173-1#02-AAM954#002 details of other certificate | [String] Declaration of Conformity | 0..* |

| idShort: | - |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | 0173-1#01-AGZ673#001 |  |  |
| Parent: | - |  |  |
| Explanation: | Note: see also [IRDI] 0112/2///61987#ABH515#003 Certificate or approval Note: CE marking is declared as mandatory according to the Blue Guide of the EU-Commission @en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] MarkingName | https://admin-shell.io/zvei/nameplate/2/0/Nameplate/Markings/Marking/MarkingName common name of the marking Note: see also [IRDI] 0173-1#02-BAB392#015 certificate/approval valueId with ECLASS enumeration IRDI is preferable, e.g. [IRDI] 0173-1#07-DAA603#004 for CE. If no IRDI available, string value can also be accepted. Note: CE marking is declared as mandatory according to Blue Guide of the EU-Commission | [String] 0173-1#07-DAA603#004 | 1 |
| [Prop] DesignationOfCertificateOrApproval | 0112/2///61987#ABH783#001 identifier of certificate, approval, declaration or license alphanumeric character sequence identifying a certificate or approval Note: Approval identifier, reference to the certificate number, to be entered without spaces | [String] Declaration of Conformity | 0..1 |
| [Prop] IssueDate | https://admin-shell.io/zvei/nameplate/2/0/Nameplate/Markings/Marking/IssueDate Date, at which the specified certificate is issued Note: format by lexical representation: CCYY-MM-DD Note: to be specified to the day | [String] 2099-01-01 | 0..1 |
| [Prop] ExpiryDate | https://admin-shell.io/zvei/nameplate/2/0/Nameplate/Markings/Marking/ExpiryDate Date, at which the specified certificate expires Note: see also ([IRDI] 0173-1#02-AAO997#001 Validity date Note: format by lexical representation: CCYY-MM-DD Note: to be specified to the day | [String] 2099-01-01 | 0..1 |
| [File] MarkingFile | https://admin-shell.io/zvei/nameplate/2/0/Nameplate/Markings/Marking/MarkingFile conformity symbol of the marking - | [File] /aasx/ce_marking.png | 1 |
| [Prop] MarkingAdditionalText | https://admin-shell.io/zvei/nameplate/2/0/Nameplate/Markings/Marking/MarkingAdditionalText where applicable, additional information on the marking in plain text, e.g. the ID-number of the notified body involved in the conformity process Note: see also [IRDI] 0173-1#02-AAM954#002 details of other certificate | [String] Declaration of Conformity | 0..* |

다. TechnicalData submodel의 하위 SubmodelCollection, Property는 다음과 같이 구성된다.

| idShort: | TechnicalData |  |  |
| --- | --- | --- | --- |
| Class: | Submodel |  |  |
| semanticId: | 0173-1#01-AHX837#002 |  |  |
| Parent: | TechnicalData |  |  |
| Explanation: | Submodel containing technical data of the asset and associated product classificatons@en,Teilmodell, das die technischen Daten des Assets und die zugehörigen Produktklassifizierungen enthält@de |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] GeneralInformation | https://admin-shell.io/ZVEI/TechnicalData/GeneralInformation/1/1 GeneralInformation General information, for example ordering and manufacturer information. - | [-] 6 elements | 1 |
| [SML] ProductClassification | 0173-1#02-ABK162#002 Product classifications Product classifications by association of product classes with common classification systems - | [-] 1 elements | 1 |
| [SMC] FurtherInformation | 0173-1#02-ABK164#002 Further information Further information on the product, the validity of the information provided and this data record - | [-] 2 elements | 1 |
| [SMC] TechnicalProperties | https://admin-shell.io/ZVEI/TechnicalData/TechnicalProperties/1/1 TechnicalProperties Technical and product properties. Individual characteristics that describe the product and its technical properties. - | [-] 2 elements | 1 |

| idShort: | GeneralInformation |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/ZVEI/TechnicalData/GeneralInformation/1/1 |  |  |
| Parent: | GeneralInformation |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] ManufacturerName | 0173-1#02-AAO677#004 Manufacturer name Legally valid designation of the natural or judicial person which is directly responsible for the design, production, packaging and labeling of a product in respect to its being brought into circulation - | [String] Example Company | 1 |
| [File] CompanyLogo | 0173-1#02-ABI776#002 Company logo Imagefile for logo of the company provided in common format (.png, .jpg) - | [File] /aasx/Logo.png | 0..1 |
| [MLP] Manufacturer ProductDesignation | 0173-1#02-AAW338#003 Manufacturer product designation Short description of the product (short text) - | [String] abcdef@en | 1 |
| [Prop] Manufacturer ArticleNumber | 0173-1#02-AAO676#005 Manufacturer article number unique product identifier of the manufacturer - | [String] A123-456 | 1 |
| [Prop] ManufacturerOrderCode | 0173-1#02-AAO227#004 Manufacturer order code by manufactures issued unique combination of numbers and letters used to identify the device for ordering - | [String] EEA-EX-200-S/47-Q3 | 1 |
| [SML] ProductImages | 0173-1#02-ABM220#001 Product images Pictorial representations of the product also related to different applications - | [-] 1 elements | 1 |

| idShort: | ProductImages |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | 0173-1#02-ABM220#001 |  |  |
| Parent: | ProductImages |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] - | 0173-1#02-ABM220#001 Product images Pictorial representations of the product also related to different applications - | [-] 2 elements | 1 |

| idShort: | - |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | 0173-1#02-ABM220#001 |  |  |
| Parent: | - |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [File] ImageFile | 0173-1#02-ABK291#002 Product image Picture of the product (low resolution) 72 dpi / rgb (GIF or JPG) - | [-] Abcde.jpg | 0..1 |
| [MLP] ImageNote | 0173-1#02-ABL423#001 Image note Notes or explanations about the image provided - | [-] Notes or explanations about the image provided@en | 0..1 |

| idShort: | ProductClassifications |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | 0173-1#02-ABK162#002 |  |  |
| Parent: | ProductClassifications |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] - | 0173-1#02-ABK162#002 Product classifications Product classifications by association of product classes with common classification systems - | [-] 7 elements | 1 |

| idShort: | - |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | 0173-1#02-ABK162#002 |  |  |
| Parent: | - |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] ClassificationSystem | 0173-1#02-ABL424#001 Classification system Reference to the scheduled collection of abstract concepts that are used to distinguish and order objects - | [String] ECLASS | 0..1 |
| [Prop] ClassificationSystemVersion | 0173-1#02-AAR710#003 Version of the classification system Referenced state of the published classification system - | [String] 15.0 | 0..1 |
| [Prop] ClassificationSystemUrl | https://admin-shell.io/IDTA/TechnicalData/ProductClassifications/ProduktClassification/ClassificationSystemUrl/2/0 Classification system URL Classification system URL - | [String] Abcde.com | 0..1 |
| [Prop] ProductClassId | 0173-1#02-ABG776#003 Product class ID Class of the associated product or industrial equipment in the classification system. According to the notation of the system (IRDI) - | [String] abcde | 0..1 |
| [Prop] ProductClassCodedName | 0173-1#02-ABK128#002 Product Class Coded Name Unique class code in a classification hierarchy - | [String] abcde | 0..1 |
| [MLP] ProductClassName | 0173-1#02-ABK273#002 Product class name Name of product class in the used classification system - | [-] Name@en | 0..1 |
| [Ref] ReferenceToTechnicalPropertyArea | 0173-1#02-ABL358#002 Reference to technical property area Reference to technical property area - | [-] abcde | 0..1 |

| idShort: | FurtherInformation |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/ZVEI/TechnicalData/FurtherInformation/1/1 |  |  |
| Parent: | FurtherInformation |  |  |
| Explanation: | Further information on the product, the validity of the information provided and this data record.@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [MLP] TextStatement | 0173-1#02-ABK134#002 Text statement Statement by the manufacturer in text form, e.g. scope of validity of the statements, scopes of application, conditions of operation. - | [-] Validity text | 0..1 |
| [Prop] ValidDate | 0173-1#02-ABL775#001 Validity date Date at which a product, process, norm or similar becomes invalid - | [Date] 2026-01-01 | 0..1 |

| idShort: | TechnicalProperties |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | 0173-1#02-ABK164#002 |  |  |
| Parent: | TechnicalProperties |  |  |
| Explanation: | Technical properties of the sintering furnace@en,소결로 기술 사양@kr |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] Manipulator | https://www.smart-factory.kr/ids/cd/Manipulator/1/0 Manipulator A collection describing the mechanical and performance characteristics of a robotic manipulator. - | [-] 7 elements | 1 |
| [SMC] Controller | https://www.smart-factory.kr/ids/cd/Controller/1/0 Controller A collection describing the type, firmware, communication, capacity, and electrical characteristics of a controller. - | [-] 6 elements | 1 |

| idShort: | Manipulator |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Manipulator/1/0 |  |  |
| Parent: | Manipulator |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] PayloadCapacity | https://www.smart-factory.kr/ids/cd/PayloadCapacity/1/0 Payload Capacity Maximum payload that the manipulator can handle under rated conditions. - | [Float] 20.0 | 0..1 |
| [Prop] AxisCount | https://www.smart-factory.kr/ids/cd/AxisCount/1/0 Axis Count Total number of controllable axes of the manipulator. - | [Int] 6 | 0..1 |
| [SML] Axis | https://www.smart-factory.kr/ids/cd/Axis/1/0 Axis A list of individual axes composing the manipulator, each with its own mechanical and motor characteristics. - | [-] 1 elements | 1 |
| [Prop] RobotwristMaximum Payload | https://www.smart-factory.kr/ids/cd/RobotwristMaximumPayload/1/0 Robot Wrist Maximum Payload Maximum payload that can be applied to the robot wrist. - | [Float] 10.0 | 0..1 |
| [Prop] Repeatability | https://www.smart-factory.kr/ids/cd/Repeatability/1/0 Repeatability Positioning repeatability of the manipulator. - | [Float] 0.05 | 0..1 |
| [Prop] Robotupperarm BackMaximum Payload | https://www.smart-factory.kr/ids/cd/RobotupperarmBackMaximumPayload/1/0 Robot Upper Arm Back Maximum Payload Maximum payload that can be applied to the back side of the robot upper arm. - | [Float] 5.0 | 0..1 |
| [Prop] MaxMotorSpeed | https://www.smart-factory.kr/ids/cd/MaxMotorSpeed/1/0 Max motor speed Maximum motor rotation speed - | [Float] 3000.0 | 0..1 |

| idShort: | Axis |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Axis/1/0 |  |  |
| Parent: | Axis |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] - | https://www.smart-factory.kr/ids/cd/AxisCollection/1/0 Axis ID Identifier assigned to an individual axis of the manipulator. - | [-] 7 elements | 1.. |

| idShort: | - |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/AxisCollection/1/0 |  |  |
| Parent: | Axis |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] AxisId | https://www.smart-factory.kr/ids/cd/AxisId/1/0 Axis ID Identifier assigned to an individual axis of the manipulator. - | [String] AXIS_01 | 0..1 |
| [Prop] MotorMaximumVibration | https://www.smart-factory.kr/ids/cd/MotorMaximumVibration/1/0 Motor Maximum Vibration Maximum permissible vibration level of the axis motor. - | [Float] 0.05 | 0..1 |
| [Prop] AxisName | https://www.smart-factory.kr/ids/cd/AxisName/1/0 Name Human-readable name of the sensor. - | [String] Base_Motor | 0..1 |
| [Prop] AxisRange | https://www.smart-factory.kr/ids/cd/AxisRange/1/0 Axis Range Mechanical motion range of the axis. - | [Float] 360.0 | 0..1 |
| [Prop] MaximumTorque | https://www.smart-factory.kr/ids/cd/MaximumTorque/1/0 Maximum Torque Maximum allowable torque that can be generated by the axis motor. - | [Float] 150.0 | 0..1 |
| [Prop] MaximumMomentofinertia | https://www.smart-factory.kr/ids/cd/MaximumMomentofinertia/1/0 Maximum Moment of Inertia Maximum allowable moment of inertia applied to the axis. - | [Float] 12.5 | 0..1 |
| [Prop] MotorMaximumTemperature | https://www.smart-factory.kr/ids/cd/MotorMaximumTemperature/1/0 Motor Maximum Temperature Maximum allowable operating temperature of the axis motor. - | [Float] 85.0 | 0..1 |

| idShort: | Controller |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Controller/1/0 |  |  |
| Parent: | Controller |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] ControllerType | https://www.smart-factory.kr/ids/cd/ControllerType/1/0 Controller Type Type or category of the controller, such as PLC, motion controller, or industrial PC. - | [String] PLC | 0..1 |
| [Prop] FirmwareVersion | https://www.smart-factory.kr/ids/cd/FirmwareVersion/1/0 Firmware Version Firmware version currently installed on the controller. - | [String] V2.1.4 | 0..1 |
| [Prop] CommunicationProtocol | https://www.smart-factory.kr/ids/cd/CommunicationProtocol/1/0 Communication Protocol Communication protocol supported or used by the controller. - | [String] EtherCAT | 0..1 |
| [Prop] InputoutputChannelQuantity | https://www.smart-factory.kr/ids/cd/InputoutputChannelQuantity/1/0 Input/Output Channel Quantity Total number of input and output channels supported by the controller. - | [Int] 32 | 0..1 |
| [Prop] MemoryCapacity | https://www.smart-factory.kr/ids/cd/MemoryCapacity/1/0 Memory Capacity Available memory capacity of the controller for program execution and data storage. - | [Float] 512.0 | 0..1 |
| [Prop] ElectricpowerSpecification | https://www.smart-factory.kr/ids/cd/ElectricpowerSpecification/1/0 Electric Power Specification Electrical power supply specification required by the controller. - | [String] 24V DC 5A | 0..1 |

라. OperationalData submodel의 하위 SubmodelCollection, Property는 다음과 같이 구성된다.

| idShort: | OperationalData |  |  |
| --- | --- | --- | --- |
| Class: | Submodel |  |  |
| semanticId: | - |  |  |
| Parent: | OperationalData |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Timestamp | https://www.smart-factory.kr/ids/cd/Timestamp/1/0 Time stamp time stamp - | [String] 2025-01-11T09:41:22Z | 1 |
| [SMC] BatchData | https://www.smart-factory.kr/ids/cd/BatchData/1/0 Batch Data Data collection of the batch. Batch-level production data collection | [-] 8 elements | 1 |
| [SMC] MonitoringData | https://www.smart-factory.kr/ids/cd/MonitoringData/1/0 Monitoring Data A collection of monitoring data - | [-] 5 elements | 1 |

| idShort: | BatchData |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/BatchData/1/0 |  |  |
| Parent: | BatchData |  |  |
| Explanation: | Batch-level production data collection@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] BatchId | https://www.smart-factory.kr/ids/cd/BatchId/1/0 Batch ID Unique identifier assigned to a production batch for traceability. Unique identifier for the production batch | [String] BATCH2026 | 0..1 |
| [Prop] StartTime | https://www.smart-factory.kr/ids/cd/StartTime/1/0 Start Time Timestamp indicating when the batch process started. Batch production start timestamp | [String] 2026-01-11T08:00:00Z | 0..1 |
| [Prop] EndTime | https://www.smart-factory.kr/ids/cd/EndTime/1/0 End Time Timestamp indicating when the batch process ended. Batch production end timestamp | [String] 2026-01-11T12:00:00Z | 0..1 |
| [Prop] BatchCumulativeEnergyUsage | https://www.smart-factory.kr/ids/cd/BatchCumulativeEnergyUsage/1/0 Batch Cumulative Energy Usage Total accumulated energy consumption during the execution of the batch. Total energy consumed during the batch | [Float] 450.5 | 0..1 |
| [Prop] UnitweightEnergyUsage | https://www.smart-factory.kr/ids/cd/UnitweightEnergyUsage/1/0 Unit Weight Energy Usage Energy consumption per unit weight of product produced during the batch. Energy consumption per unit weight of product | [Float] 2.25 | 0..1 |
| [SMC] Setting | https://www.smart-factory.kr/ids/cd/Setting/1/0 Setting A collection of configurable process parameter setpoints used to control equipment operation. Equipment setting parameters | [-] 3 elements | 0..1 |
| [SMC] ProductionInformation | https://www.smart-factory.kr/ids/cd/ProductionInformation/1/0 Production Information A collection of production-related identifiers and key figures, including order, lot, product information, quantities, and downtime records. Production order and output information | [-] 8 elements | 0..1 |
| [SMC] Threshold | https://www.smart-factory.kr/ids/cd/Threshold/1/0 Threshold A collection of upper and lower threshold values used for process monitoring and control. Operational limit threshold settings | [-] 4 elements | 0..1 |

| idShort: | Setting |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Setting/1/0 |  |  |
| Parent: | Setting |  |  |
| Explanation: | Equipment setting parameters@en,장비 설정 파라미터@ko |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] PickingPosition | https://www.smart-factory.kr/ids/cd/PickingPosition/1/0 Ejector Initial Position Picking position value of the robot arm Initial position of the robot arm | [String] X: 120.5, Y: 45.0, Z: 10.2 | 0..1 |
| [Prop] GripPosition | https://www.smart-factory.kr/ids/cd/GripPosition/1/0 Grip Position Target position value set for the robot during operation. grip position of the robot | [String] X: 120.5, Y: 45.0, Z: 5.0 | 0..1 |
| [Prop] MotorSpeed | https://www.smart-factory.kr/ids/cd/MotorSpeed/1/0 Motor Speed Configured rotational speed of the motor. Motor rotation speed setting | [Float] 1500 | 0..1 |

| idShort: | ProductionInformation |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/ProductionInformation/1/0 |  |  |
| Parent: | ProductionInformation |  |  |
| Explanation: | Production order and output information@en,생산 주문 및 산출 정보@ko |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] OrderNumber | https://www.smart-factory.kr/ids/cd/OrderNumber/1/0 Order Number Identifier of the production order associated with the current production record. Production order number | [String] PO-2026-00458 | 0..1 |
| [Prop] LotNumber | https://www.smart-factory.kr/ids/cd/LotNumber/1/0 Lot Number Identifier of the production lot for traceability. Production lot number | [String] LOT-2026-0211-B2 | 0..1 |
| [Prop] ProductType | https://www.smart-factory.kr/ids/cd/ProductType/1/0 Product Type Product type or category associated with the current production record. Type classification of the product | [String] Green Body Type A | 0..1 |
| [Prop] ProductCode | https://www.smart-factory.kr/ids/cd/ProductCode/1/0 Product Code Unique product code or identifier used for production and traceability. Product identification code | [String] PCODE-GB-X100 | 0..1 |
| [Prop] ProductionQuantity | https://www.smart-factory.kr/ids/cd/ProductionQuantity/1/0 Production Quantity Quantity of products produced within the current production record. Number of units produced | [int] 500 | 0..1 |
| [Prop] TotalOperationTime | https://www.smart-factory.kr/ids/cd/TotalOperationTime/1/0 Total Operation Time Accumulated operating time of the equipment for the current production record. Total equipment operation time | [Float] 4.5 | 0..1 |
| [Prop] DowntimeCause | https://www.smart-factory.kr/ids/cd/DowntimeCause/1/0 Downtime Cause Cause or reason code describing why downtime occurred. Reason for production downtime | [String] Scheduled Maintenance | 0..1 |
| [Prop] Downtime | https://www.smart-factory.kr/ids/cd/Downtime/1/0 Downtime Accumulated downtime duration for the current production record. Duration of production downtime | [Float] 15.0 | 0..1 |

| idShort: | Threshold |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Threshold/1/0 |  |  |
| Parent: | Threshold |  |  |
| Explanation: | Operational limit threshold settings@en,운전 한계 임계값 설정@ko |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] EjectorAxesUpperlimit | https://www.smart-factory.kr/ids/cd/EjectorAxesUpperlimit/1/0 Ejector Axes Upper Limit Maximum allowable position limit of the ejector axis. Upper limit of ejector axis position | [Float] 150.0 | 0..1 |
| [Prop] EjectorAxesLowerlimit | https://www.smart-factory.kr/ids/cd/EjectorAxesLowerlimit/1/0 Ejector Axes Lower Limit Minimum allowable position limit of the ejector axis. Lower limit of ejector axis position | [Float] 0.0 | 0..1 |
| [Prop] SpeedUpperlimit | https://www.smart-factory.kr/ids/cd/SpeedUpperlimit/1/0 Speed Upper Limit Maximum allowable operational speed limit. Upper limit of operation speed | [Float] 500.0 | 0..1 |
| [Prop] SpeedLowerlimit | https://www.smart-factory.kr/ids/cd/SpeedLowerlimit/1/0 Speed Lower Limit Minimum allowable operational speed limit. Lower limit of operation speed | [Float] 10.0 | 0..1 |

| idShort: | MonitoringData |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/MonitoringData/1/0 |  |  |
| Parent: | MonitoringData |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] AlarmStatus | https://www.smart-factory.kr/ids/cd/AlarmStatus/1/0 Alarm Status Status indicating whether a robot-related alarm is active or cleared. - | [String] Active | 0..1 |
| [Prop] AlarmCode | https://www.smart-factory.kr/ids/cd/AlarmCode/1/0 Alarm Code Code identifying the specific robot-related alarm condition. - | [String] MATERR005 | 0..1 |
| [SMC] EnvironmentInformation | https://www.smart-factory.kr/ids/cd/EnvironmentInformation/1/0 Environment Information A collection of environmental measurement values describing the ambient conditions around the equipment. Environmental condition measurements | [-] 3 elements | 0..1 |
| [SMC] EquipmentStatus | https://www.smart-factory.kr/ids/cd/EquipmentStatus/1/0 Equipment Status A collection of equipment status Equipment operational status information | [-] 8 elements | 0..1 |
| [SMC] Sensorhealth | https://www.smart-factory.kr/ids/cd/Sensorhealth/1/0 Sensor Health A collection of metadata and status information describing the health condition of a sensor. - | [-] 4 elements | 0..1 |

| idShort: | EnvironmentInformation |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/EnvironmentInformation/1/0 |  |  |
| Parent: | EnvironmentInformation |  |  |
| Explanation: | Environmental condition measurements@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] MeasurementTemperature | https://www.smart-factory.kr/ids/cd/MeasurementTemperature/1/0 Measurement Temperature Measured ambient temperature in the environment surrounding the equipment. - | [Float] 24.5 | 0..1 |
| [Prop] MeasurementHumidity | https://www.smart-factory.kr/ids/cd/MeasurementHumidity/1/0 Measurement Humidity Measured relative humidity of the ambient environment. - | [Float] 45.0 | 0..1 |
| [Prop] MeasurementNoiseLevel | https://www.smart-factory.kr/ids/cd/MeasurementNoiseLevel/1/0 Measurement Noise Level Measured ambient noise level in the environment surrounding the equipment. - | [Float] 65.2 | 0..1 |

| idShort: | EquipmentStatus |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/EquipmentStatus/1/0 |  |  |
| Parent: | EquipmentStatus |  |  |
| Explanation: | Equipment operational status information@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] OperationType | https://www.smart-factory.kr/ids/cd/OperationType/1/0 Operation Type Type of operation currently executed by the equipment, such as automatic, manual, or maintenance mode. - | [String] Automatic | 0..1 |
| [Prop] PowerStatus | https://www.smart-factory.kr/ids/cd/PowerStatus/1/0 Power Status Status indicating whether the equipment power is on, off, or in a standby state. - | [String] On | 0..1 |
| [Prop] SafetyInterlockStatus | https://www.smart-factory.kr/ids/cd/SafetyInterlockStatus/1/0 Safety Interlock Status Status of the safety interlock system indicating whether safety conditions are satisfied. - | [String] Satisfied | 0..1 |
| [Prop] OperationAvailabilityStatus | https://www.smart-factory.kr/ids/cd/OperationAvailabilityStatus/1/0 Operation Availability Status Status indicating whether the equipment is available for operation or not. - | [String] Available | 0..1 |
| [Prop] RecipeInformation | https://www.smart-factory.kr/ids/cd/RecipeInformation/1/0 Recipe Information Identifier or reference to the recipe configuration currently applied to the equipment. - | [String] RECIPESTV3 | 0..1 |
| [SMC] RobotSystem | https://www.smart-factory.kr/ids/cd/RobotSystem/1/0 Robot System A collection representing system-level status information of a robot manipulator. Robot manipulator system status | [-] 0 elements | 0..1 |
| [SMC] MotorInformation | https://www.smart-factory.kr/ids/cd/MotorInformation/1/0 Motor Information A collection of identification, operational status, health condition, and alarm-related information associated with a motor. - | [-] 8 elements | 0..1 |
| [SMC] ConsumableInformation | https://www.smart-factory.kr/ids/cd/ConsumableInformation/1/0 Consumable Information A collection of identifiers and descriptive information related to consumable items used by the equipment. - | [-] 4 elements | 0..1 |

| idShort: | MotorInformation |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/MotorInformation/1/0 |  |  |
| Parent: | MotorInformation |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] MotorId | https://www.smart-factory.kr/ids/cd/MotorId/1/0 Motor ID Identifier of the motor associated with the heater. Unique identifier of the motor | [String] MTR-Z1-004 | 0..1 |
| [Prop] AlarmMessage | https://www.smart-factory.kr/ids/cd/AlarmMessage/1/0 Alarm Message Human-readable alarm message related to motor handling or motor status. Alarm description message | [String] Motor Overload | 0..1 |
| [Prop] OperationStatus | https://www.smart-factory.kr/ids/cd/OperationStatus/1/0 Operation Status Current operational state of the heater, such as running, stopped, or standby. Motor operation status | [String] Running | 0..1 |
| [Prop] HealthStatus | https://www.smart-factory.kr/ids/cd/HealthStatus/1/0 Health Status Overall health condition of the motor based on diagnostics or monitoring results. Motor health condition status | [String] Good | 0..1 |
| [Prop] AbnormalityDetectionStatus | https://www.smart-factory.kr/ids/cd/AbnormalityDetectionStatus/1/0 Abnormality Detection Status Status indicating whether an abnormal condition has been detected in the motor. Abnormality detection flag | [String] Detected | 0..1 |
| [Prop] AbnormalityCause | https://www.smart-factory.kr/ids/cd/AbnormalityCause/1/0 Abnormality Cause Description or code identifying the cause of a detected abnormality in the motor. Cause of detected abnormality | [String] ERROVERHEAT | 0..1 |
| [Prop] AlarmStatus | https://www.smart-factory.kr/ids/cd/AlarmStatus/1/0 Alarm Status Status indicating whether a motor-related alarm is active or cleared. Alarm activation status | [String] Active | 0..1 |
| [Prop] AlarmCode | https://www.smart-factory.kr/ids/cd/AlarmCode/1/0 Alarm Code Code identifying the specific motor-related alarm condition. Alarm identification code | [String] ALMSHUTDOWN | 0..1 |

| idShort: | ConsumableInformation |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/ConsumableInformation/1/0 |  |  |
| Parent: | ConsumableInformation |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] ConsumableID | https://www.smart-factory.kr/ids/cd/ConsumableID/1/0 Consumable ID Unique identifier assigned to a consumable item for tracking and management. - | [String] GAS-N2-REG-001 | 0..1 |
| [Prop] ConsumableDescription | https://www.smart-factory.kr/ids/cd/ConsumableDescription/1/0 Consumable Description Textual description providing details about the consumable item. - | [String] High-purity Nitrogen gas | 0..1 |
| [Prop] ConsumableName | https://www.smart-factory.kr/ids/cd/ConsumableName/1/0 Consumable Name Human-readable name of the consumable item. - | [String] Nitrogen Gas | 0..1 |
| [Prop] ReplacementDate | https://www.smart-factory.kr/ids/cd/ReplacementDate/1/0 Replacement Date Date or timestamp indicating when the consumable item was last replaced. - | [String] 2026-01-10T14:00:00Z | 0..1 |

| idShort: | Sensorhealth |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Sensorhealth/1/0 |  |  |
| Parent: | Sensorhealth |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] SensorID | https://www.smart-factory.kr/ids/cd/SensorID/1/0 ID Unique identifier of the sensor within the system. - | [String] SENSOR-TEMP-Z1-001 | 0..1 |
| [Prop] SensorStatus | https://www.smart-factory.kr/ids/cd/SensorStatus/1/0 Status Current health status of the sensor (e.g., OK, Warning, Fault, Offline). - | [String] OK | 0..1 |
| [Prop] SensorName | https://www.smart-factory.kr/ids/cd/SensorName/1/0 Name Human-readable name of the sensor. - | [String] Chamber Upper Temperature Sensor | 0..1 |
| [Prop] SensorDescription | https://www.smart-factory.kr/ids/cd/SensorDescription/1/0 Description Additional description providing context about the sensor or its status. - | [String] Thermocouple for monitoring | 0..1 |

마. HandoverDocumentation submodel의 하위 SubmodelCollection, Property는 다음과 같이 구성된다.

| idShort: | HandoverDocumentation |  |  |
| --- | --- | --- | --- |
| Class: | Submodel |  |  |
| semanticId: | 0173-1#01-AHF578#003 |  |  |
| Parent: | HandoverDocumentation |  |  |
| Explanation: | The Submodel defines a set meta data for the handover of documentation from the manufacturer to the operator for industrial equipment@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SML] Documents | 0173-1#02-ABI500#003 Documents (handover documentation) Information for a document identity - | [-] 1 elements | 1 |
| [SML] Entities | https://admin-shell.io/vdi/2770/1/0/EntitiesForDocumentation Entities Entities for documentation - | [-] 1 elements | 0..1 |

| idShort: | Documents |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | 0173-1#02-ABI500#003 |  |  |
| Parent: | Documents |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] - | 0173-1#02-ABI500#003 Documents (handover documentation) Information for a document identity This SubmodelElementCollection holds the information for a VDI 2770 Document entity | [-] 4 elements | 1..* |

| idShort: | - |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | 0173-1#02-ABI500#003 |  |  |
| Parent: | - |  |  |
| Explanation: | This SubmodelElementCollection holds the information for a VDI 2770 Document entity@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SML] DocumentIds | 0173-1#02-ABI501#003 Document identifiers Set of document identifiers for the document. One ID in this collection should be used as a preferred ID - | [-] 1 elements | 1 |
| [SML] DocumentClassifications | 0173-1#02-ABI502#003 Document classification Information about a document classification entity - | [-] 1 elements | 1 |
| [SML] DocumentVersions | 0173-1#02-ABI503#003 DocumentVersions Information elements of individual Document Version entities - | [-] 1 elements | 1 |
| [SML] DocumentedEntities | https://admin-shell.io/vdi/2770/1/0/Document/DocumentedEntities Documented Entities Documented Entities - | [-] 1 elements | 0..1 |

| idShort: | DocumentIds |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | 0173-1#02-ABI501#003 |  |  |
| Parent: | DocumentIds |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] - | 0173-1#02-ABI501#003 Document identifiers Set of document identifiers for the document. One ID in this collection should be used as a preferred ID This SubmodelElementCollection holds the information for a VDI 2770 Document entity | [-] 3 elements | 1 |

| idShort: | - |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | 0173-1#02-ABI501#003 |  |  |
| Parent: | - |  |  |
| Explanation: | This SubmodelElementCollection holds the information for a VDI 2770 Document entity@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] DocumentDomainId | 0173-1#02-ABH994#003 Document domain id Identification of the domain in which the given DocumentId is unique. The domain ID can e.g., be the name or acronym of the providing organisation - | [String] https://domain.com | 1 |
| [Prop] DocumentIdentifier | 0173-1#02-AAO099#004 DocumentIdentifier alphanumeric character sequence uniquely identifying a document - | [String] XF90-884 | 1 |
| [Prop] DocumentIsPrimary | 0173-1#02-ABH995#003 DocumentIsPrimary Flag indicating that a DocumentId within a collection of at least two DocumentId`s is the ‘primary’ identifier for the document. This is the preferred ID of the document (commonly from the point of view of the owner of the asset) - | [Boolean] true | 0..1 |

| idShort: | DocumentClassifications |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | 0173-1#02-ABI502#003 |  |  |
| Parent: | DocumentClassifications |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] - | 0173-1#02-ABI502#003 Document classification Information about a document classification entity Set of information for describing the classification of the Document according to a ClassificationSystem | [-] 3 elements | 1..* |

| idShort: | - |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | 0173-1#02-ABI502#003 |  |  |
| Parent: | - |  |  |
| Explanation: | Set of information for describing the classification of the Document according to a ClassificationSystem@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] ClassId | 0173-1#02-ABH996#003 Class identifyer Unique ID of the document class within a classficationsystem - | [String] 03-02 | 1 |
| [MLP] ClassName | 0173-1#02-ABJ219#002 Class name Name of the class in the classification system - | [String] Operation@en | 1 |
| [Prop] ClassificationSystem | 0173-1#02-ABH997#003 Classification system Identification of the classification system - | [String] VDI2770:2020 | 1 |

| idShort: | DocumentVersions |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | 0173-1#02-ABI503#003 |  |  |
| Parent: | DocumentVersions |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] - | 0173-1#02-ABI503#003 DocumentVersions Information elements of individual Document Version entities - | [-] 15 elements | 1..* |

| idShort: | - |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | 0173-1#02-ABI503#003 |  |  |
| Parent: | - |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SML] Language | 0173-1#02-AAN468#008 Language of document Language style of the document - | [Property] en | 1 |
| [Prop] Version | 0173-1#02-AAP003#005 Document version Design that partly deviates from the previous - | [String] V1.2 | 1 |
| [MLP] Title | 0173-1#02-ABG940#003 Document name Name of the document - | [String] Examplary title@en | 1 |
| [MLP] Subtitle | 0173-1#02-ABH998#003 Subtitle List of language-dependent subtitles of the document - | [String] Examplary subtitle@en | 0..1 |
| [MLP] Description | 0173-1#02-AAN466#004 Document description Plain text characterizing the content of the document - | [String] Abstract@en | 1 |
| [MLP] KeyWords | 0173-1#02-ABH999#003 Keywords List of language-dependent keywords of the document - | [String] Examplary keywords@en | 0..1 |
| [Prop] StatusSetDate | 0173-1#02-ABI000#003 Document status set date Date when the document status was set - | [Date] 2020-02-06 | 1 |
| [Prop] StatusValue | 0173-1#02-ABI001#003 Document status Each document version represents a point in time in the document life cycle. This status value refers to the milestones in the document life cycle. The following two values should be used for the application of this guideline: InReview (under review), Released (released) - | [String] Released | 1 |
| [Prop] OrganizationShortName | 0173-1#02-ABI002#003 Organization Short Name Short name of the organization - | [String] Example company | 1 |
| [Prop] OrganizationOfficialName | 0173-1#02-ABI004#003 Organization official name Official name of the organization of the author of the document - | [String] Example company Ltd. | 1 |
| [SML] RefersToEntities | 0173-1#02-ABK288#002 Reference to other documents Forms a generic refers to-relationship to another document or document version. They have a loose relationship - | [-] | 0..1 |
| [SML] BasedOnReferences | 0173-1#02-ABK289#002 Based on other documents Forms a based on-relationship to another document or document version. Typically states, that the content of the document bases on another document (e.g. specification requirements). Both have a strong relationship - | [-] | 0..1 |
| [SML] TranslationOfEntities | 0173-1#02-ABK290#002 Translation of other document Forms a translation of-relationship to another document or document version. Both have a strong relationship - | [-] | 0..1 |
| [SML] DigitalFiles | 0173-1#02-ABK126#002 Digital file MIME-Type, file name and file contents given by the file SubmodelElement - | [-] | 1 |
| [File] PreviewFile | 0173-1#02-ABK127#002 Preview file Provides a preview image of the Document Version, e.g. first page, in a commonly used image format and low resolution - | [File] *.jpg | 0..1 |

| idShort: | Language |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | 0173-1#02-AAN468#008 |  |  |
| Parent: | Language |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] - | 0173-1#02-AAN468#008 Language of document Language style of the document - | [String] en | 1 |

| idShort: | RefersToEntities |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | 0173-1#02-ABK288#002 |  |  |
| Parent: | RefersToEntities |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Ref] - | 0173-1#02-ABK288#002 Reference to other documents Forms a generic refers to-relationship to another document or document version. They have a loose relationship - | [-] | 1..* |

| idShort: | BasedOnReferences |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | 0173-1#02-ABK289#002 |  |  |
| Parent: | BasedOnReferences |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Ref] - | 0173-1#02-ABK289#002 Based on other documents Forms a based on-relationship to another document or document version. Typically states, that the content of the document bases on another document (e.g. specification requirements). Both have a strong relationship - | [-] | 1..* |

| idShort: | TranslationOfEntities |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | 0173-1#02-ABK290#002 |  |  |
| Parent: | TranslationOfEntities |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Ref] - | 0173-1#02-ABK290#002 Translation of other document Forms a translation of-relationship to another document or document version. Both have a strong relationship - | [-] | 1..* |

| idShort: | DigitalFiles |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | 0173-1#02-ABK126#002 |  |  |
| Parent: | DigitalFiles |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [File] - | 0173-1#02-ABK126#002 Digital file MIME-Type, file name and file contents given by the file SubmodelElement - | [-] | 1..* |

| idShort: | DocumentedEntities |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | https://admin-shell.io/vdi/2770/1/0/Document/DocumentedEntities |  |  |
| Parent: | DocumentedEntities |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Ref] - | https://admin-shell.io/vdi/2770/1/0/Document/DocumentedEntity DocumentedEntity Reference for the documented entities - | [-] | 1..* |

| idShort: | Entities |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | https://admin-shell.io/vdi/2770/1/0/EntitiesForDocumentation |  |  |
| Parent: | Entities |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Ent] - | https://admin-shell.io/vdi/2770/1/0/EntityForDocumentation Entity States that the described Entity is an important entity for documentation of the superordinate Asset of the Asset Administration Shell. - | [-] - | 1..* |

바. PredictiveMaintenance submodel의 하위 SubmodelCollection, Property는 다음과 같이 구성된다.

| idShort: | PredictiveMaintenance |  |  |
| --- | --- | --- | --- |
| Class: | Submodel |  |  |
| semanticId: | https://admin-shell.io/idta/SubmodelTemplate/PredictiveMaintenance/1/0 |  |  |
| Parent: | PredictiveMaintenance |  |  |
| Explanation: | The Submodel Predictive Maintenance is a collection of properties to provide information for predictive maintenance use cases. It is intended to use this submodel in sub-systems of production lines to describe predictive maintenance relevant topics for the sub-system, as well as to use this submodel in predictive maintenance software applications@en,Das Teilmodell Predictive Maintenance stellt Informationen für Anwendungsfälle von Predictive Maintenance bereit. Dieses Teilmodell ist für Sub-Systeme von Produktionslinien vorgesehen, um Predictive Maintenance relevante Informationen für das Sub-System zu beschreiben sowie für Software-Anwendungen für vorausschauende Wartung@de |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] RemainingUsfulLifePrediction | https://admin-shell.io/idta/PredictiveMaintenance/RemainingUsefulLifePrediction/1/0 Remaining useful life prediction Information about remaining useful life (RUL) prediction in the context of predictive maintenance Information about remaining useful life (RUL) prediction in the context of predictive maintenance | [-] 7 elements | 0..1 |

| idShort: | RemainingUsefulLifePrediction |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/PredictiveMaintenance/RemainingUsefulLifePrediction/1/0 |  |  |
| Parent: | RemainingUsefulLifePrediction |  |  |
| Explanation: | Information about remaining useful life (RUL) prediction in the context of predictive maintenance@en,Informationen über die Prognose der nutzbaren Restlebensdauer (RUL) im Kontext der vorausschauenden Wartung@de |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Ent] RemainingUsefulLifetime | https://admin-shell.io/idta/PredictiveMaintenance/RemainingUsefulLifetime/1/0 RemainingUsefulLifetime Life time Remaining useful life time, bades on Lifetime model of OPC Foundation (https://reference.opcfoundation.org/DI/v104/docs/10) | [-] - | 0..1 |
| [Prop] RemainingUsfulLifeDateTime | https://admin-shell.io/idta/PredictiveMaintenance/RemainingUsfulLifeDateTime/1/0 time stamp (date and time) mark attributed to an instant by means of a specified timescale, expressed as a date and a time mark attributed to an instant by means of a specified timescale, expressed as a date and a time | [DateTime] 2026-06-30T18:00:00+09:00 | 0..1 |
| [Range] ConfidenceInterval | https://admin-shell.io/idta/PredictiveMaintenance/ConfidenceInterval/1/0 Confidence interval confidence interval, measured in the unit of the predicted value confidence interval, measured in the unit of the predicted value | [-] 0.85 0.95 | 0..1 |
| [SML] ListRULBoundaryConditions | https://admin-shell.io/idta/PredictiveMaintenance/ListRULBoundaryConditions/1/0 List of boundary conditions of remaining useful life prediction List of boundary conditions for which remaining useful life has been predicted List of boundary conditions for which remaining useful life has been predicted | [-] 1 elements | 0..1 |
| [SMC] PredictionModelInformation | https://admin-shell.io/idta/PredictiveMaintenance/PredictionModelInformation/1/0 Prediction model information Information about the model for RUL prediction relevant in the context of predictive maintenance Information about the model for RUL prediction relevant in the context of predictive maintenance | [-] 3 elements | 0..1 |
| [SML] ListPreAlerts | https://www.smart-factory.kr/ids/cd/ListPreAlerts/1/0 ListPreAlerts List for defining pre-alerts which should be raised before remaining useful life is exceeded List for defining pre-alerts which should be raised before remaining useful life is exceeded | [-] 1 elements | 0..1 |
| [SMC] AlertAfterExceedingRemainingUsableLife | https://admin-shell.io/idta/PredictiveMaintenance/AlertAfterExceedingRemainingUsableLife/1/0 Alert after exceeding remaining useful life Alert information which should be triggered or displayed after exceeding remaining useful life Alert information which should be triggered or displayed after exceeding remaining useful life | [-] 2 elements | 0..1 |

| idShort: | RemainingUsefulLifetime |  |  |
| --- | --- | --- | --- |
| Class: | Entity |  |  |
| semanticId: | https://admin-shell.io/idta/PredictiveMaintenance/RemainingUsefulLifetime/1/0 |  |  |
| Parent: | RemainingUsefulLifetime |  |  |
| Explanation: | Remaining useful life time, bades on Lifetime model of OPC Foundation (https://reference.opcfoundation.org/DI/v104/docs/10)@en,Verbleibende Betriebszeit in Anlehnung an Lifetime model der OPC Foundation (https://reference.opcfoundation.org/DI/v104/docs/10)@de |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] IndicationType | https://admin-shell.io/idta/PredictiveMaintenance/IndicationType/1/0 Type of duration Type of wear-relevant duration, e.g. time, cycles, distance, etc. Type of wear-relevant duration, e.g. time, cycles, distance, etc. | [String] time | 0..1 |
| [Prop] DurationValue | https://admin-shell.io/idta/PredictiveMaintenance/DurationValue/1/0 Duration value Value of duration in the wear relevant unit, e.g. time, operation cycles, distance, etc. Value of duration in the wear relevant unit, e.g. time, operation cycles, distance, etc. | [Double] 720.0 | 0..1 |
| [Prop] EngineeringUnit | https://admin-shell.io/idta/PredictiveMaintenance/EngineeringUnit/1/0 Unit of the duration Wear relevant physical unit, e.g. time, operation cycles, distance, etc. Wear relevant physical unit, e.g. time, operation cycles, distance, etc. | [String] h | 0..1 |
| [Prop] StartValue | https://admin-shell.io/idta/PredictiveMaintenance/StartValue/1/0 Start value of duration Starting value from which the duration is measured in the wear-relevant unit, e.g. time, cycles, distance, etc. Starting value from which the duration is measured in the wear-relevant unit, e.g. time, cycles, distance, etc. | [Double] 0.0 | 0..1 |
| [Prop] StartDateTime | https://admin-shell.io/idta/PredictiveMaintenance/StartDateTime/1/0 Start datetime of the duration Start date and time from which the duration is measured Start date and time from which the duration is measured | [DateTime] 2026-02-01T00:00:00+09:00 | 0..1 |
| [Prop] Description | https://admin-shell.io/idta/PredictiveMaintenance/Description/1/0 Description of the duration Description of the wear duration information Description of the wear duration information | [String] RUL | 0..1 |

| idShort: | ListRULBoundaryConditions |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | https://admin-shell.io/idta/PredictiveMaintenance/ListRULBoundaryConditions/1/0 |  |  |
| Parent: | ListRULBoundaryConditions |  |  |
| Explanation: | List of boundary conditions for which remaining useful life has been predicted@en,Liste der Randbedingungen unter denen die nutzbare Restlebensdauer prognostiziert wurde@de |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] - | https://admin-shell.io/idta/PredictiveMaintenance/RULCondition/1/0 Boundary condition for remaining useful life prediction Boundary condition for which remaining useful life has been predicted Boundary condition for which remaining useful life has been predicted | [-] 9 elements | 1 |

| idShort: | - |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/PredictiveMaintenance/RULCondition/1/0 |  |  |
| Parent: | - |  |  |
| Explanation: | Boundary condition for which remaining useful life has been predicted@en,Randbedingung unter der die nutzbare Restlebensdauer prognositiziert wurde@de |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] ConditionName | 0173-1#02-AAV958#001 StatusConditionName Name of the asset's state, highlighting deviations from normal operations or reduced lifespan. Name of the indicator (process value, KPI, material property, asset property) describing a boundary condition for which RUL prediction is valid. | [String] BearingTemperature | 0..1 |
| [Prop] IsBoundaryConditionUsedInModel | https://admin-shell.io/idta/PredictiveMaintenance/IsBoundaryConditionUsedInModel/1/0 Condition is used in RUL model If this boundary condition is used in the model for RUL prediction true, else false If this boundary condition is used in the model for RUL prediction true, else false | [Boolean] true | 0..1 |
| [Prop] BoundaryValueRange | https://admin-shell.io/idta/PredictiveMaintenance/BoundaryValueRange/1/0 Indicator value range Value range of the indicator (process value, KPI, material property, asset property) describing a boundary condition for which RUL prediction is valid. Value range of the indicator (process value, KPI, material property, asset property) describing a boundary condition for which RUL prediction is valid. | [Double] 60.0 | 0..1 |
| [Prop] BoundaryEngineeringUnit | https://admin-shell.io/idta/PredictiveMaintenance/BoundaryEngineeringUnit/1/0 Unit of the indicator Engineering Unit of the boundary condition indicator Engineering Unit of the boundary condition indicator | [String] degC | 0..1 |
| [Range] DriftInfoAIModel | https://admin-shell.io/idta/PredictiveMaintenance/DriftInfoAIModel/1/0 Drift information Drift information of AI model Drift information of AI model | [-] -0.02 0.03 | 0..1 |
| [Prop] MeanValue | https://admin-shell.io/idta/PredictiveMaintenance/MeanValue/1/0 Mean value Mean value of the distribution of indicator values Mean value of the distribution of indicator values | [Double] 70.0 | 0..1 |
| [Prop] Standarddeviation | https://admin-shell.io/idta/PredictiveMaintenance/Standarddeviation/1/0 Standard deviation Standard deviation of the distribution of indicator values Standard deviation of the distribution of indicator values | [Double] 5.0 | 0..1 |
| [Prop] Skewness | https://admin-shell.io/idta/PredictiveMaintenance/Skewness/1/0 Skewness Skewness of the distribution of indicator values Skewness of the distribution of indicator values | [Double] 0.1 | 0..1 |
| [Prop] BoundaryDescription | https://admin-shell.io/idta/PredictiveMaintenance/BoundaryDescription/1/0 Description of the boundary of a RUL prediction condition Description of the boundary of a RUL prediction condition Description of the boundary of a RUL prediction condition | [String] Valid operating range under which the remaining useful life prediction is applicable | 0..1 |

| idShort: | PredictionModelInformation |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/PredictiveMaintenance/PredictionModelInformation/1/0 |  |  |
| Parent: | PredictionModelInformation |  |  |
| Explanation: | Information about the model for RUL prediction relevant in the context of predictive maintenance@en,Informationen über das Modell zur Prognose der RUL im Kontext der vorausschauenden Wartung@de |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] ModelType | https://admin-shell.io/idta/PredictiveMaintenance/ModelType/1/0 Model type Model type as an enumerated value: physical based methods, data-driven methods, hybrid methods Model type as an enumerated value: physical based methods, data-driven methods, hybrid methods | [String] data-driven methods | 0..1 |
| [Prop] ModelDescription | https://admin-shell.io/idta/PredictiveMaintenance/ModelDescription/1/0 Model description More detailed description of the model type used for RUL prediction (optional) More detailed description of the model type used for RUL prediction (optional) | [String] Data-driven model | 0..1 |
| [Ref] SMAIModelNamePlate | https://admin-shell.io/idta/PredictiveMaintenance/SMAIModelNamePlate/1/0 Reference to AIModelNameplate Reference to AIModelNameplate Reference to AIModelNameplate | [-] | 0..1 |

| idShort: | ListPreAlerts |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/ListPreAlerts/1/0 |  |  |
| Parent: | ListPreAlerts |  |  |
| Explanation: | List for defining pre-alerts which should be raised before remaining useful life is exceeded@en,Liste, um Voralarme zu definieren, die vor Erreichen der Restlebensdauer ausgelöst werden sollen@de |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] - | https://www.smart-factory.kr/ids/cd/PreAlerts/1/0 PreAlerts Defining pre-alerts which should be raised before remaining useful life is exceeded Definition of a pre-alert which should be raised before remaining useful life is exceeded | [-] 2 elements | 1 |

| idShort: | - |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/PreAlerts/1/0 |  |  |
| Parent: | - |  |  |
| Explanation: | Definition of a pre-alert which should be raised before remaining useful life is exceeded@en,Definition eiens Voralarms, der vor Erreichen der Restlebensdauer ausgelöst werden sollen@de |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] PreAlertMessage | https://admin-shell.io/idta/PredictiveMaintenance/PreAlertMessage/1/0 Remaining usable life alert message Message to be displayed when alarm regarding predicted remaining useful life is raised Message to be displayed when alarm regarding predicted remaining useful life is raised | [String] RUL is approaching the predefined threshold | 0..1 |
| [Prop] PreAlertValue | https://admin-shell.io/idta/PredictiveMaintenance/PreAlertValue/1/0 Pre-alert value Pre-warning duration in wear relevant unit before remaining useful life is exceeded Pre-warning duration in wear relevant unit before remaining useful life is exceeded | [Double] 72.0 | 0..1 |

| idShort: | AlertAfterExceedingRemainingUsableLife |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/PredictiveMaintenance/AlertAfterExceedingRemainingUsableLife/1/0 |  |  |
| Parent: | AlertAfterExceedingRemainingUsableLife |  |  |
| Explanation: | Alert information which should be triggered or displayed after exceeding remaining useful life@en,Alarminformationen die ausgelöst oder angezeigt werden sollen, wenn die Restlebensdauer überschritten wird@de |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] AlertMessage | https://admin-shell.io/idta/PredictiveMaintenance/AlertMessage/1/0 Remaining usable life alert message Message to be displayed when alarm regarding predicted remaining useful life is raised Message to be displayed when alarm regarding predicted remaining useful life is raised | [String] Maintenance required | 0..1 |
| [Prop] MaintenanceRequired | https://admin-shell.io/idta/PredictiveMaintenance/MaintenanceRequired/1/0 Maintenance required Maintenance required Maintenance required | [Boolean] true | 0..1 |

사. AIModelNameplate submodel의 하위 SubmodelCollection, Property는 다음과 같이 구성된다.

| idShort: | AIModelNameplate |  |  |
| --- | --- | --- | --- |
| Class: | Submodel |  |  |
| semanticId: | - |  |  |
| Parent: | AIModelNameplate |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] URIOfTheProduct | 0173-1#02-AAY811#001 URI of the product unique global identification of the product using an universal resource identifier (URI) unique global identification of the product instance using an universal resource identifier (URI) | [String] https://www.smart-factory.kr/ids/aas/abc/1/0 | 0..1 |
| [Prop] Version | 0173-1#02-AAS354#002 Version Version Version of the nameplate | [String] 1.0 | 0..1 |
| [Ref] ContactInformation | https://admin-shell.io/idta/AIModelNameplate/Contactinformation/1/0 ContactInformation contactInformation references Reference to the Contact Information IDTA Submodel to describe the responsible person for the Submodel | [-] | 0..1 |
| [Prop] Storage | https://admin-shell.io/idta/AIModelNameplate/Storage/1/0 Storage path to the model(e.g. local path, serverpath,...) Path to the model (e.g. local path, serverpath,...) | [String] local | 0..1 |
| [MLP] KindOfLearning | https://admin-shell.io/idta/AIModelNameplate/KindOfLearning/1/0 KindOfLearning This property describes the learningmethod of the model (supervised, unsupervised, reinforcement, ....) This property describes the learningmethod of the model (supervised, unsupervised, reinforcement, ....) | [-] reinforced@en | 0..1 |
| [SMC] Inputs | https://admin-shell.io/idta/AIModelNameplate/Inputs/1/0 Inputs This SMC collects necessary information about the Modelinput Collection of necessary information about the model input | [-] 3 elements | 0..1 |
| [SMC] Outputs | https://admin-shell.io/idta/AIModelNameplate/Outputs/1/0 Outputs This SMC collects necassary information about the Modeloutput. Collection of necessary information about the model output | [-] 1 elements | 0..1 |
| [SMC] TrainingResults | https://admin-shell.io/idta/AIModelNameplate/TrainingResults/1/0 TrainingResults This SMC holds different trainingresults of the model. Collection of different training results of the model | [-] 1 elements | 0..1 |
| [SMC] Plots | https://admin-shell.io/idta/AIModelNameplate/Plots/1/0 Plots This SMC is used to save and manage images of different plots e.g. about the training or the model itself. Collection of different plots | [-] 3 elements | 0..1 |
| [SMC] Details | https://admin-shell.io/idta/AIModelNameplate/Details/1/0 Details This SMC is used to document additional details about the model. Collection of additional details about the model | [-] 4 elements | 0..1 |
| [SMC] AIDataset | https://admin-shell.io/idta/AIModelNameplate/AIDataset/1/0 AI Dataset Dataset of AI Collection of additional details about the model | [-] 2 elements | 0..1 |
| [SMC] AITypeSpecificInformation | https://admin-shell.io/idta/AIModelNameplate/AITypeSpecificInformation/1/0 AITypeSpecificInformation This collection contains different kinds of AI-typespecific parameters. Collection containing different kinds of AI method specific parameters | [-] 3 elements | 0..1 |

| idShort: | Inputs |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/AIModelNameplate/Inputs/1/0 |  |  |
| Parent: | Inputs |  |  |
| Explanation: | Collection of necessary information about the model input@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] KindOfInput | https://admin-shell.io/idta/AIModelNameplate/Inputs/KindOfInput/1/0 KindOfInput Kind of data used for input (e.g. Images, Audio) Type of data (e.g. Images, Audio) | [String] | 0..1 |
| [SMC] DimensionN | https://admin-shell.io/idta/AIModelNameplate/Inputs/DimensionN/1/0 DimensionN This SMC contains Information about every inputdimension. It exists one collection for every inputdimension. Collection of information about a single input dimension | [-] 2 elements | 0..1 |
| [File] Preprocessing | https://admin-shell.io/idta/AIModelNameplate/Inputs/Preprocessing/1/0 Preprocessing This property can be used to submit your prepocessing pipeline as a file (e.g directly as code or just the used transforms and parameters) Preprocessing pipeline as a file | [-] pipeline_v1.json | 0..1 |

| idShort: | DimensionN |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/AIModelNameplate/Inputs/DimensionN/1/0 |  |  |
| Parent: | DimensionN |  |  |
| Explanation: | Collection of information about a single input dimension@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Size | https://admin-shell.io/idta/AIModelNameplate/Inputs/DimensionN/Size/1/0 Size This property holds the size of this dimension. Size of the output dimension | [String] 1 | 0..1 |
| [Prop] Information | https://admin-shell.io/idta/AIModelNameplate/Inputs/DimensionN/Information/1/0 Information This property holds information about the usage of this dimension. Information about the usage of this dimension | [String] Single scalrar input | 0..1 |

| idShort: | Outputs |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/AIModelNameplate/Outputs/1/0 |  |  |
| Parent: | Outputs |  |  |
| Explanation: | Collection of necessary information about the model output@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] DimensionN | https://admin-shell.io/idta/AIModelNameplate/Outputs/DimensionN/1/0 DimensionN This is a example SMC. It holds information about every outputdimension. replace the "N" with the dimension this SMC is for. Collection of information about a single output dimension | [-] 2 elements | 1 |

| idShort: | DimensionN |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/AIModelNameplate/Outputs/DimensionN/1/0 |  |  |
| Parent: | DimensionN |  |  |
| Explanation: | Collection of information about a single output dimension@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Size | https://admin-shell.io/idta/AIModelNameplate/Outputs/DimensionN/Size/1/0 Size This property holds the size of this dimension. Size of the output dimension | [String] 1 | 0..1 |
| [MLP] Result | https://admin-shell.io/idta/AIModelNameplate/Outputs/DimensionN/Result/1/0 Result This property describes the meaning of this outputdimension (e.g. class). Meaning of the output dimension (e.g. class) | [-] class @en | 0..1 |

| idShort: | TrainingResults |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/AIModelNameplate/TrainingResults/1/0 |  |  |
| Parent: | TrainingResults |  |  |
| Explanation: | Collection of different training results of the model@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] ExampleResult | https://admin-shell.io/idta/AIModelNameplate/TrainingResults/Result/1/0 ExampleResult This property is a placeholder example for a result for the trained ai. Result of the trained AI | [String] Predicted remaining useful life: 720 h | 1 |

| idShort: | Plots |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/AIModelNameplate/Plots/1/0 |  |  |
| Parent: | Plots |  |  |
| Explanation: | Collection of different plots @en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] TrainingTesting | https://admin-shell.io/idta/AIModelNameplate/Plots/TrainingTesting/1/0 TrainingTesting This SMC contains plots about the training and testing of the model. Collection of plots about training results and test results | [-] 1 elements | 0..1 |
| [SMC] Structure | https://admin-shell.io/idta/AIModelNameplate/Plots/Structure/1/0 Structure This SMC contains images about the structure (e.g. of a neural net). Collection describing the model architecture (e.g. of a neural net) | [-] 1 elements | 0..1 |
| [SMC] GeneralPlots | https://admin-shell.io/idta/AIModelNameplate/Plots/GeneralPlots/1/0 GeneralPlots This SMC can be used to save plots about other things not fitting in the other two SMCs. Collection of plots not fitting in existing collections | [-] 1 elements | 0..1 |

| idShort: | TrainingTesting |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/AIModelNameplate/Plots/TrainingTesting/1/0 |  |  |
| Parent: | TrainingTesting |  |  |
| Explanation: | Collection of plots about training results and test results@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [File] ExamplePlot | https://admin-shell.io/idta/AIModelNameplate/Plots/TrainingTesting/Plot/1/0 ExamplePlot This is a simple file example for a plotimage. Training or test result plot | [-] | 1 |

| idShort: | Structure |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/AIModelNameplate/Plots/Structure/1/0 |  |  |
| Parent: | Structure |  |  |
| Explanation: | Collection describing the model architecture (e.g. of a neural net)@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [File] Example StructurePlot | https://admin-shell.io/idta/AIModelNameplate/Plots/Structure/StructurePlot/1/0 ExampleStructurePlot This is a placeholder example for a structure image. Image of a model architecture | [-] | 1 |

| idShort: | GeneralPlots |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/AIModelNameplate/Plots/GeneralPlots/1/0 |  |  |
| Parent: | GeneralPlots |  |  |
| Explanation: | Collection of plots not fitting in existing collections@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [File] ExamplePlot | https://admin-shell.io/idta/AIModelNameplate/Plots/GeneralPlots/Plot/1/0 ExamplePlot1 Placerholder for other plots. Image of an model related plot | [-] | 1 |

| idShort: | Details |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/AIModelNameplate/Details/1/0 |  |  |
| Parent: | Details |  |  |
| Explanation: | Collection of additional details about the model@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] FileExtension | https://admin-shell.io/idta/AIModelNameplate/Details/DataEnding/1/0 FileExtension This property tells the dataending it got saved with (e.g. .pth). File extension of the model (e.g. .pth) | [String] .pth | 0..1 |
| [Prop] AIFramework | https://admin-shell.io/idta/AIModelNameplate/Details/AIFramework/1/0 AIFramework This property describes the AI framework and its version (e.g. pytorch 11.3). AI framework and its version (e.g. pytorch 11.3) | [String] PyTorch 2.0 | 0..1 |
| [Prop] ProgramLanguage | https://admin-shell.io/idta/AIModelNameplate/Details/ProgramLanguage/1/0 ProgramLanguage The used programming language and version (e.g. python 3.9). Programming language and version (e.g. python 3.9) | [String] Python 3.9 | 0..1 |
| [File] Requirements | https://admin-shell.io/idta/AIModelNameplate/Details/Requirements/1/0 Requirements This component is used to save the requirements.txt of the enviroment. Requirements file containing the required software dependencies (e.g. Python Requirements.txt file) | [-] | 0..1 |

| idShort: | AIDataset |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/AIModelNameplate/AIDataset/1/0 |  |  |
| Parent: | AIDataset |  |  |
| Explanation: | Collection of additional details about the model@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Ref] AIDatasetReference | https://admin-shell.io/idta/AIModelNameplate/AIDataset/DatasetReference/1/0 DatasetReference Dataset reference Reference to the used AIDataset-Submodel | [-] | 1 |
| [Range] TimeStamp | https://admin-shell.io/idta/AIModelNameplate/AITypeSpecificInformation/NeuralNetwork/TransferLearning/TimeStamp/1/0 TimeStamp Because there is a possibility, that the origin model has changed its weights or structure over time, it is necessary to date the time this model was used for transfer learning Determining the used data, when dataset differs over time | [-] 2025-01-01T00:00:00Z 2025-12-31T23:59:59Z | 1 |

| idShort: | AITypeSpecificInformation |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/AIModelNameplate/AITypeSpecificInformation/1/0 |  |  |
| Parent: | AITypeSpecificInformation |  |  |
| Explanation: | Collection containing different kinds of AI method specific parameters@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] TransferLearning | https://admin-shell.io/idta/AIModelNameplate/AITypeSpecificInformation/TransferLearning/1/0 TransferLearning This SMC is used to describe informations, if the model is a result of transfer learning, of the origin model Collection containing information of the origin model, if the model is a result of transfer learning | [-] 4 elements | 0..1 |
| [SMC] Hyperparameter | https://admin-shell.io/idta/AIModelNameplate/AITypeSpecificInformation/Hyperparameter/1/0 Hyperparameter This SMC contains the relevant hyperparamter used for train the NN. Collection containing relevant hyperparamter | [-] 1 elements | 0..1 |
| [MLP] Type | https://admin-shell.io/idta/AIModelNameplate/AITypeSpecificInformation/Type/1/0 Type Type of the AI(e.g. CNN, RNN, LSTM, ...) Type of the AI(e.g. CNN, RNN, LSTM, ...) | [-] CNN@en | 0..1 |

| idShort: | TransferLearning |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/AIModelNameplate/AITypeSpecificInformation/TransferLearning/1/0 |  |  |
| Parent: | TransferLearning |  |  |
| Explanation: | Collection containing information of the origin model, if the model is a result of transfer learning@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Name | https://admin-shell.io/idta/AIModelNameplate/AITypeSpecificInformation/NeuralNetwork/TransferLearning/Name/1/0 Name Holds the name of the origin NN-Model (e.g. resnet18) Name of the origin neural network (e.g. resnet18) | [String] Resnet18 | 0..1 |
| [Prop] TimeStamp | https://admin-shell.io/idta/AIModelNameplate/AITypeSpecificInformation/NeuralNetwork/TransferLearning/TimeStamp/1/0 TimeStamp Because there is a possibility, that the origin model has changed its weights or structure over time, it is necessary to date the time this model was used for transfer learning Date of access of the origin neural network | [DateTime] 2025-01-01T00:00:00Z | 0..1 |
| [File] Weights | https://admin-shell.io/idta/AIModelNameplate/AITypeSpecificInformation/NeuralNetwork/TransferLearning/Weights/1/0 Weights Possibility to save the weights of the origin model as a file (e.g. .txt) File containing the weights of the origin model (e.g. .txt) | [-] | 0..1 |
| [Ref] AIModelNameplate | https://admin-shell.io/idta/AIModelNameplate/AITypeSpecificInformation/NeuralNetwork/TransferLearning/ModelnameplateRef/1/0 ModelNameplate This is a reference to the model used for transfer learning if there exists a submodel to it. Reference to the AI Modelnameplate submodel of the origin neural network | [-] | 0..1 |

| idShort: | Hyperparameter |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://admin-shell.io/idta/AIModelNameplate/AITypeSpecificInformation/Hyperparameter/1/0 |  |  |
| Parent: | Hyperparameter |  |  |
| Explanation: | Collection containing relevant hyperparamter@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Example | https://admin-shell.io/idta/AIModelNameplate/AITypeSpecificInformation/Hyperparameter/LearningRate/1/0 Hyperparameter Hyperparameter of AI Example hyperparameter (e.g. learning rate of the model) | [String] Learning rate | 1 |

아. AssetInterfacesDescription submodel의 하위 SubmodelCollection, Property는 다음과 같이 구성된다.

| idShort: | AssetInterfacesDescription |  |  |
| --- | --- | --- | --- |
| Class: | Submodel |  |  |
| semanticId: | - |  |  |
| Parent: | AssetInterfacesDescription |  |  |
| Explanation: | AID Template Sample@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] InterfaceTemplateForHttp | https://www.smart-factory.kr/ids/cd/InterfaceTemplateForHttp/1/0 Interface template for HTTP A template defining the HTTP-based communication interface for the asset, including RESTful API endpoints. - | [-] 7 elements | 0..1 |
| [SMC] InterfaceTemplateForModbus | https://www.smart-factory.kr/ids/cd/InterfaceTemplateForMODBUS/1/0 Interface template for MODBUS A template defining the Modbus-based communication interface for industrial automation and control systems. - | [-] 7 elements | 0..1 |
| [SMC] InterfaceTemplateForMqtt | https://www.smart-factory.kr/ids/cd/InterfaceTemplateForMQTT/1/0 Interface template for MQTT A template defining the MQTT-based publish-subscribe communication interface for IoT and real-time messaging. - | [-] 7 elements | 0..1 |

| idShort: | InterfaceTemplateForHttp |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/InterfaceTemplateForHttp/1/0 |  |  |
| Parent: | InterfaceTemplateForHttp |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Title | https://www.smart-factory.kr/ids/cd/Title/1/0 Title A human-readable name or title that provides a short description of the interface or element. - | [String] Asset Interface Description | 0..1 |
| [Prop] Created | https://www.smart-factory.kr/ids/cd/Created/1/0 Created The date and time when the interface description was initially created. - | [DateTime] 2026-02-01T10:00:00+09:00 | 0..1 |
| [Prop] Modified | https://www.smart-factory.kr/ids/cd/Modified/1/0 Modified The date and time when the interface description was last modified. - | [DateTime] 2026-02-04T09:30:00+09:00 | 0..1 |
| [Prop] Support | https://www.smart-factory.kr/ids/cd/Support/1/0 Support Contact information or URI for obtaining support related to the interface. - | [String] https://www.smart-factory.kr | 0..1 |
| [SMC] EndpointMetadata | https://www.smart-factory.kr/ids/cd/EndpointMetadata/1/0 Endpoint metadata Metadata describing the communication endpoint, including base URI, content type, and security configurations. - | [-] 4 elements | 0..1 |
| [SMC] InteractionMetadata | https://www.smart-factory.kr/ids/cd/InteractionMetadata/1/0 Interaction metadata Metadata describing the available interactions with the asset, including properties, actions, and events. - | [-] 3 elements | 0..1 |
| [SMC] ExternalDescriptor | https://www.smart-factory.kr/ids/cd/ExternalDescriptor/1/0 External descriptor A reference to an external file containing additional interface description, such as a WoT Thing Description. - | [-] 1 elements | 0..1 |

| idShort: | EndpointMetadata |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/EndpointMetadata/1/0 |  |  |
| Parent: | EndpointMetadata |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Base | https://www.smart-factory.kr/ids/cd/Base/1/0 Base The base URI that provides the root address for all relative URIs in the interface. - | [String] https://www.smart-factory.kr | 0..1 |
| [Prop] ContentType | https://www.smart-factory.kr/ids/cd/ContentType/1/0 Content type The MIME type indicating the format of the data payload, such as application/json or text/xml. - | [String] Text/xml | 0..1 |
| [SML] Security | https://www.smart-factory.kr/ids/cd/Security/1/0 Security Reference to the security configuration that applies to the interface or interaction. - | [-] 1 elements | 0..1 |
| [SMC] SecurityDefinitions | https://www.smart-factory.kr/ids/cd/SecurityDefinitions/1/0 Security definitions A collection of security scheme definitions that can be referenced by security configurations. - | [-] 9 elements | 0..1 |

| idShort: | Security |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Security/1/0 |  |  |
| Parent: | Security |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Ref] - | https://www.smart-factory.kr/ids/cd/Security/1/0 Reference to the security configuration that applies to the interface or interaction. | [-] | 1 |

| idShort: | SecurityDefinitions |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/SecurityDefinitions/1/0 |  |  |
| Parent: | SecurityDefinitions |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] NosecSc | https://www.smart-factory.kr/ids/cd/NosecSc/1/0 No security scheme A security scheme indicating that no authentication or security mechanism is required. - | [-] 1 elements | 0..1 |
| [SMC] AutoSc | https://www.smart-factory.kr/ids/cd/AutoSc/1/0 Auto security scheme A security scheme where the security mechanism is automatically determined by the protocol or context. - | [-] 2 elements | 0..1 |
| [SMC] BasicSc | https://www.smart-factory.kr/ids/cd/BasicSc/1/0 Basic security scheme A security scheme using HTTP Basic Authentication with username and password credentials. - | [-] 4 elements | 0..1 |
| [SMC] ComboSc | https://www.smart-factory.kr/ids/cd/ComboSc/1/0 Combo security scheme A security scheme that combines multiple security schemes using logical AND or OR operations. - | [-] 4 elements | 0..1 |
| [SMC] ApikeySc | https://www.smart-factory.kr/ids/cd/ApikeySc/1/0 API key security scheme A security scheme using an API key for authentication, typically passed in a header or query parameter. - | [-] 4 elements | 0..1 |
| [SMC] PskSc | https://www.smart-factory.kr/ids/cd/PskSc/1/0 Pre-shared key security scheme A security scheme using a pre-shared key (PSK) for authentication, commonly used in CoAP and DTLS. - | [-] 3 elements | 0..1 |
| [SMC] DigestSc | https://www.smart-factory.kr/ids/cd/DigestSc/1/0 Digest security scheme A security scheme using HTTP Digest Authentication with challenge-response mechanism. - | [-] 5 elements | 0..1 |
| [SMC] BearerSc | https://www.smart-factory.kr/ids/cd/BearerSc/1/0 Bearer security scheme A security scheme using bearer tokens, typically JWT or OAuth2 access tokens. - | [-] 7 elements | 0..1 |
| [SMC] Oauth2Sc | https://www.smart-factory.kr/ids/cd/Oauth2Sc/1/0 OAuth2 security scheme A security scheme using OAuth 2.0 protocol for authorization and access control. - | [-] 7 elements | 0..1 |

| idShort: | NosecSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/NosecSc/1/0 |  |  |
| Parent: | NosecSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] apikey | 1 |

| idShort: | AutoSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/AutoSc/1/0 |  |  |
| Parent: | AutoSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] Oauth2 | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | BasicSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/BasicSc/1/0 |  |  |
| Parent: | BasicSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Name | https://www.smart-factory.kr/ids/cd/Name/1/0 Name Human-readable name of the sensor. - | [String] Authorization | 0..1 |
| [Prop] In | https://www.smart-factory.kr/ids/cd/In/1/0 In The location where the security credential is expected, such as header, query, body, or cookie. - | [String] header | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | ComboSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/ComboSc/1/0 |  |  |
| Parent: | ComboSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [SML] OneOf | https://www.smart-factory.kr/ids/cd/OneOf/1/0 One of A list of security schemes where at least one must be satisfied (logical OR combination). - | [-] 0 elements | 0..1 |
| [SML] AllOf | https://www.smart-factory.kr/ids/cd/AllOf/1/0 All of A list of security schemes where all must be satisfied (logical AND combination). - | [-] 0 elements | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | ApikeySc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/ApikeySc/1/0 |  |  |
| Parent: | ApikeySc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Name | https://www.smart-factory.kr/ids/cd/Name/1/0 Name Human-readable name of the sensor. - | [String] Authorization | 0..1 |
| [Prop] In | https://www.smart-factory.kr/ids/cd/In/1/0 In The location where the security credential is expected, such as header, query, body, or cookie. - | [String] header | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | PskSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/PskSc/1/0 |  |  |
| Parent: | PskSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Identity | https://www.smart-factory.kr/ids/cd/Identity/1/0 Identity The identity hint used to identify the pre-shared key in PSK security schemes. - | [String] examplekey | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | DigestSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/DigestSc/1/0 |  |  |
| Parent: | DigestSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Name | https://www.smart-factory.kr/ids/cd/Name/1/0 Name Human-readable name of the sensor. - | [String] Authorization | 0..1 |
| [Prop] In | https://www.smart-factory.kr/ids/cd/In/1/0 In The location where the security credential is expected, such as header, query, body, or cookie. - | [String] header | 0..1 |
| [Prop] Qop | https://www.smart-factory.kr/ids/cd/Qop/1/0 Quality of protection The quality of protection value for digest authentication, such as auth or auth-int. - | [String] auth | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | BearerSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/BearerSc/1/0 |  |  |
| Parent: | BearerSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Name | https://www.smart-factory.kr/ids/cd/Name/1/0 Name Human-readable name of the sensor. - | [String] Authorization | 0..1 |
| [Prop] In | https://www.smart-factory.kr/ids/cd/In/1/0 In The location where the security credential is expected, such as header, query, body, or cookie. - | [String] header | 0..1 |
| [Prop] Authorization | https://www.smart-factory.kr/ids/cd/Authorization/1/0 Authorization The URI of the authorization server for obtaining access tokens. - | [String] https://auth.com/token | 0..1 |
| [Prop] Alg | https://www.smart-factory.kr/ids/cd/Alg/1/0 Algorithm The cryptographic algorithm used for signing or encrypting tokens, such as ES256 or RS256. - | [String] RS256 | 0..1 |
| [Prop] Format | https://www.smart-factory.kr/ids/cd/Format/1/0 Format The format of the security token, such as jwt, cwt, or jwe. - | [String] jwt | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | Oauth2Sc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Oauth2Sc/1/0 |  |  |
| Parent: | Oauth2Sc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Token | https://www.smart-factory.kr/ids/cd/Token/1/0 Token The URI of the token endpoint for obtaining access tokens in OAuth2 flows. - | [String] https://auth.com/token | 0..1 |
| [Prop] Refresh | https://www.smart-factory.kr/ids/cd/Refresh/1/0 Refresh The URI of the endpoint for refreshing expired access tokens using a refresh token. - | [String] https://auth.com/token | 0..1 |
| [Prop] Authorization | https://www.smart-factory.kr/ids/cd/Authorization/1/0 Authorization The URI of the authorization server for obtaining access tokens. - | [String] https://auth.com/token | 0..1 |
| [Prop] Scopes | https://www.smart-factory.kr/ids/cd/Scopes/1/0 Scopes The OAuth2 scopes that define the access permissions requested or granted. - | [String] read | 0..1 |
| [Prop] Flow | https://www.smart-factory.kr/ids/cd/Flow/1/0 Flow The OAuth2 authorization flow type, such as code, client, or device. - | [String] Client | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | InteractionMetadata |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/InteractionMetadata/1/0 |  |  |
| Parent: | InteractionMetadata |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] Properties | https://www.smart-factory.kr/ids/cd/Properties/1/0 Properties A collection of property affordances that expose the readable and writable state of the asset. - | [-] 1 elements | 0..1 |
| [SMC] Actions | https://www.smart-factory.kr/ids/cd/Actions/1/0 Actions A collection of action affordances that can be invoked to perform operations on the asset. - | [-] 0 elements | 0..1 |
| [SMC] Events | https://www.smart-factory.kr/ids/cd/Events/1/0 Events A collection of event affordances that describe asynchronous notifications from the asset. - | [-] 0 elements | 0..1 |

| idShort: | Properties |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Properties/1/0 |  |  |
| Parent: | Properties |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] PropertyName | https://www.smart-factory.kr/ids/cd/PropertyName/1/0 Property name The name identifier of a property within an object schema definition. Current counter value | [-] 13 elements | 1 |

| idShort: | PropertyName |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/PropertyName/1/0 |  |  |
| Parent: | PropertyName |  |  |
| Explanation: | Current counter value@en,Derzeitiger Zählerwert@de,Valore attuale del contatore@it |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Key | https://www.smart-factory.kr/ids/cd/Key/1/0 Key A unique identifier or key used to reference a specific element or value. | [String] temp | 0..1 |
| [Prop] Type | https://www.smart-factory.kr/ids/cd/Type/1/0 Type The data type of a value, such as string, integer, number, boolean, object, or array. | [String] number | 0..1 |
| [Prop] Title | https://www.smart-factory.kr/ids/cd/Title/1/0 Title A human-readable name or title that provides a short description of the interface or element. | [String] temp | 0..1 |
| [Prop] Observable | https://www.smart-factory.kr/ids/cd/Observable/1/0 Observable Indicates whether the property supports observation for value change notifications. | [Boolean] true | 0..1 |
| [Prop] Const | https://www.smart-factory.kr/ids/cd/Const/1/0 Constant A fixed constant value that the property must always have. | [Int] 1 | 0..1 |
| [Prop] Default | https://www.smart-factory.kr/ids/cd/Default/1/0 Default The default value used when no explicit value is provided. | [String] 0 | 0..1 |
| [Prop] Unit | https://www.smart-factory.kr/ids/cd/Unit/1/0 Unit The unit of measurement for the property value, typically expressed as a UN/CEFACT unit code. | [String] degC | 0..1 |
| [Range] MinMax | https://www.smart-factory.kr/ids/cd/min_max/1/0 min max value | [-] 0 100 | 0..1 |
| [Range] LengthRange | https://www.smart-factory.kr/ids/cd/lengthRange/1/0 length range value | [-] 0 100 | 0..1 |
| [SMC] Items | https://www.smart-factory.kr/ids/cd/Items/1/0 Items The schema definition for items contained within an array type property. | [-] 9 elements | 0..1 |
| [Range] ItemsRange | https://www.smart-factory.kr/ids/cd/itemsRange/1/0 items range value | [-] 0 100 | 0..1 |
| [Ref] ValueSemantics | https://www.smart-factory.kr/ids/cd/ValueSemantics/1/0 Value semantics Semantic annotation providing the meaning or context of the property value. | [-] | 0..1 |
| [SMC] Forms | https://www.smart-factory.kr/ids/cd/Forms/1/0 Forms A collection of form definitions that describe how to access or invoke the interaction affordance. | [-] 5 elements | 0..1 |

| idShort: | Items |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Items/1/0 |  |  |
| Parent: | Items |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Type | https://www.smart-factory.kr/ids/cd/Type/1/0 Type The data type of a value, such as string, integer, number, boolean, object, or array. - | [String] number | 1 |
| [Prop] Unit | https://www.smart-factory.kr/ids/cd/Unit/1/0 Unit The unit of measurement for the property value, typically expressed as a UN/CEFACT unit code. - | [String] degC | 0..1 |
| [Prop] Default | https://www.smart-factory.kr/ids/cd/Default/1/0 Default The default value used when no explicit value is provided. - | [String] 0 | 0..1 |
| [Prop] Const | https://www.smart-factory.kr/ids/cd/Const/1/0 Constant A fixed constant value that the property must always have. - | [Int] 1 | 0..1 |
| [Prop] Observable | https://www.smart-factory.kr/ids/cd/Observable/1/0 Observable Indicates whether the property supports observation for value change notifications. - | [Boolean] true | 0..1 |
| [Prop] Title | https://www.smart-factory.kr/ids/cd/Title/1/0 Title A human-readable name or title that provides a short description of the interface or element. - | [String] temp | 0..1 |
| [Range] MinMax | https://www.smart-factory.kr/ids/cd/min_max/1/0 min max value | [-] 0 100 | 0..1 |
| [Range] LengthRange | https://www.smart-factory.kr/ids/cd/lengthRange/1/0 length range value | [-] 0 100 | 0..1 |
| [Ref] ValueSemantics | https://admin-shell.io/idta/AssetInterfacesDescription/1/0/valueSemantics - - - | [-] | 0..1 |

| idShort: | Forms |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Forms/1/0 |  |  |
| Parent: | Forms |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Href | https://www.smart-factory.kr/ids/cd/Href/1/0 Hyperlink reference The target URI or relative path for accessing the interaction affordance. - | [String] https://Href.com | 0..1 |
| [Prop] ContentType | https://www.smart-factory.kr/ids/cd/ContentType/1/0 Content type The MIME type indicating the format of the data payload, such as application/json or text/xml. - | [String] text | 0..1 |
| [SML] Security | https://www.smart-factory.kr/ids/cd/Security/1/0 Security Reference to the security configuration that applies to the interface or interaction. - | [-] 1 elements | 0..1 |
| [Prop] HtvMethodName | https://www.smart-factory.kr/ids/cd/HtvMethodName/1/0 HTTP method name The HTTP method to be used for the request, such as GET, POST, PUT, or DELETE. - | [String] GET | 0..1 |
| [SMC] HtvHeaders | https://www.smart-factory.kr/ids/cd/HtvHeaders/1/0 HTTP headers A collection of HTTP header fields to be included in the request. - | [-] 2 elements | 0..1 |

| idShort: | Security |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Security/1/0 |  |  |
| Parent: | Security |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Ref] - | https://www.smart-factory.kr/ids/cd/Security/1/0 Reference to the security configuration that applies to the interface or interaction. | [-] | 1 |

| idShort: | HtvHeaders |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/HtvHeaders/1/0 |  |  |
| Parent: | HtvHeaders |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] HtvFieldName | https://www.smart-factory.kr/ids/cd/HtvFieldName/1/0 HTTP field name The name of an HTTP header field. | [String] Authorization | 0..1 |
| [Prop] HtvFieldValue | https://www.smart-factory.kr/ids/cd/HtvFieldValue/1/0 HTTP field value The value of an HTTP header field. - | [String] Bearer <access_token> | 0..1 |

| idShort: | ExternalDescriptor |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/ExternalDescriptor/1/0 |  |  |
| Parent: | ExternalDescriptor |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [File] FileName | https://www.smart-factory.kr/ids/cd/FileName/1/0 File name The name or path of the external descriptor file. - | [-] | 1 |

| idShort: | InterfaceTemplateForModbus |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/InterfaceTemplateForMODBUS/1/0 |  |  |
| Parent: | InterfaceTemplateForModbus |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Title | https://www.smart-factory.kr/ids/cd/Title/1/0 Title A human-readable name or title that provides a short description of the interface or element. - | [String] Asset Interface Description | 0..1 |
| [Prop] Created | https://www.smart-factory.kr/ids/cd/Created/1/0 Created The date and time when the interface description was initially created. - | [DateTime] 2026-02-01T10:00:00+09:00 | 0..1 |
| [Prop] Modified | https://www.smart-factory.kr/ids/cd/Modified/1/0 Modified The date and time when the interface description was last modified. - | [DateTime] 2026-02-04T09:30:00+09:00 | 0..1 |
| [Prop] Support | https://www.smart-factory.kr/ids/cd/Support/1/0 Support Contact information or URI for obtaining support related to the interface. - | [String] https://www.smart-factory.kr | 0..1 |
| [SMC] EndpointMetadata | https://www.smart-factory.kr/ids/cd/EndpointMetadata/1/0 Endpoint metadata Metadata describing the communication endpoint, including base URI, content type, and security configurations. - | [-] 4 elements | 0..1 |
| [SMC] InteractionMetadata | https://www.smart-factory.kr/ids/cd/InteractionMetadata/1/0 Interaction metadata Metadata describing the available interactions with the asset, including properties, actions, and events. - | [-] 3 elements | 0..1 |
| [SMC] ExternalDescriptor | https://www.smart-factory.kr/ids/cd/ExternalDescriptor/1/0 External descriptor A reference to an external file containing additional interface description, such as a WoT Thing Description. - | [-] 1 elements | 0..1 |

| idShort: | EndpointMetadata |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/EndpointMetadata/1/0 |  |  |
| Parent: | EndpointMetadata |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Base | https://www.smart-factory.kr/ids/cd/Base/1/0 Base The base URI that provides the root address for all relative URIs in the interface. - | [String] https://www.smart-factory.kr | 0..1 |
| [Prop] ContentType | https://www.smart-factory.kr/ids/cd/ContentType/1/0 Content type The MIME type indicating the format of the data payload, such as application/json or text/xml. - | [String] Text/xml | 0..1 |
| [SML] Security | https://www.smart-factory.kr/ids/cd/Security/1/0 Security Reference to the security configuration that applies to the interface or interaction. - | [-] 1 elements | 0..1 |
| [SMC] SecurityDefinitions | https://www.smart-factory.kr/ids/cd/SecurityDefinitions/1/0 Security definitions A collection of security scheme definitions that can be referenced by security configurations. - | [-] 9 elements | 0..1 |

| idShort: | Security |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Security/1/0 |  |  |
| Parent: | Security |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Ref] - | https://www.smart-factory.kr/ids/cd/Security/1/0 Reference to the security configuration that applies to the interface or interaction. | [-] | 1 |

| idShort: | SecurityDefinitions |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/SecurityDefinitions/1/0 |  |  |
| Parent: | SecurityDefinitions |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] NosecSc | https://www.smart-factory.kr/ids/cd/NosecSc/1/0 No security scheme A security scheme indicating that no authentication or security mechanism is required. - | [-] 1 elements | 0..1 |
| [SMC] AutoSc | https://www.smart-factory.kr/ids/cd/AutoSc/1/0 Auto security scheme A security scheme where the security mechanism is automatically determined by the protocol or context. - | [-] 2 elements | 0..1 |
| [SMC] BasicSc | https://www.smart-factory.kr/ids/cd/BasicSc/1/0 Basic security scheme A security scheme using HTTP Basic Authentication with username and password credentials. - | [-] 4 elements | 0..1 |
| [SMC] ComboSc | https://www.smart-factory.kr/ids/cd/ComboSc/1/0 Combo security scheme A security scheme that combines multiple security schemes using logical AND or OR operations. - | [-] 4 elements | 0..1 |
| [SMC] ApikeySc | https://www.smart-factory.kr/ids/cd/ApikeySc/1/0 API key security scheme A security scheme using an API key for authentication, typically passed in a header or query parameter. - | [-] 4 elements | 0..1 |
| [SMC] PskSc | https://www.smart-factory.kr/ids/cd/PskSc/1/0 Pre-shared key security scheme A security scheme using a pre-shared key (PSK) for authentication, commonly used in CoAP and DTLS. - | [-] 3 elements | 0..1 |
| [SMC] DigestSc | https://www.smart-factory.kr/ids/cd/DigestSc/1/0 Digest security scheme A security scheme using HTTP Digest Authentication with challenge-response mechanism. - | [-] 5 elements | 0..1 |
| [SMC] BearerSc | https://www.smart-factory.kr/ids/cd/BearerSc/1/0 Bearer security scheme A security scheme using bearer tokens, typically JWT or OAuth2 access tokens. - | [-] 7 elements | 0..1 |
| [SMC] Oauth2Sc | https://www.smart-factory.kr/ids/cd/Oauth2Sc/1/0 OAuth2 security scheme A security scheme using OAuth 2.0 protocol for authorization and access control. - | [-] 7 elements | 0..1 |

| idShort: | NosecSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/NosecSc/1/0 |  |  |
| Parent: | NosecSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] apikey | 1 |

| idShort: | AutoSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/AutoSc/1/0 |  |  |
| Parent: | AutoSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] Oauth2 | 1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 1 |

| idShort: | BasicSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/BasicSc/1/0 |  |  |
| Parent: | BasicSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Name | https://www.smart-factory.kr/ids/cd/Name/1/0 Name Human-readable name of the sensor. - | [String] Authorization | 0..1 |
| [Prop] In | https://www.smart-factory.kr/ids/cd/In/1/0 In The location where the security credential is expected, such as header, query, body, or cookie. - | [String] header | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | ComboSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/ComboSc/1/0 |  |  |
| Parent: | ComboSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [SML] OneOf | https://www.smart-factory.kr/ids/cd/OneOf/1/0 One of A list of security schemes where at least one must be satisfied (logical OR combination). - | [-] 0 elements | 0..1 |
| [SML] AllOf | https://www.smart-factory.kr/ids/cd/AllOf/1/0 All of A list of security schemes where all must be satisfied (logical AND combination). - | [-] 0 elements | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | ApikeySc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/ApikeySc/1/0 |  |  |
| Parent: | ApikeySc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Name | https://www.smart-factory.kr/ids/cd/Name/1/0 Name Human-readable name of the sensor. - | [String] Authorization | 0..1 |
| [Prop] In | https://www.smart-factory.kr/ids/cd/In/1/0 In The location where the security credential is expected, such as header, query, body, or cookie. - | [String] header | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | PskSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/PskSc/1/0 |  |  |
| Parent: | PskSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Identity | https://www.smart-factory.kr/ids/cd/Identity/1/0 Identity The identity hint used to identify the pre-shared key in PSK security schemes. - | [String] examplekey | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | DigestSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/DigestSc/1/0 |  |  |
| Parent: | DigestSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Name | https://www.smart-factory.kr/ids/cd/Name/1/0 Name Human-readable name of the sensor. - | [String] Authorization | 0..1 |
| [Prop] In | https://www.smart-factory.kr/ids/cd/In/1/0 In The location where the security credential is expected, such as header, query, body, or cookie. - | [String] header | 0..1 |
| [Prop] Qop | https://www.smart-factory.kr/ids/cd/Qop/1/0 Quality of protection The quality of protection value for digest authentication, such as auth or auth-int. - | [String] auth | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | BearerSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/BearerSc/1/0 |  |  |
| Parent: | BearerSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Name | https://www.smart-factory.kr/ids/cd/Name/1/0 Name Human-readable name of the sensor. - | [String] Authorization | 0..1 |
| [Prop] In | https://www.smart-factory.kr/ids/cd/In/1/0 In The location where the security credential is expected, such as header, query, body, or cookie. - | [String] header | 0..1 |
| [Prop] Authorization | https://www.smart-factory.kr/ids/cd/Authorization/1/0 Authorization The URI of the authorization server for obtaining access tokens. - | [String] https://auth.com/token | 0..1 |
| [Prop] Alg | https://www.smart-factory.kr/ids/cd/Alg/1/0 Algorithm The cryptographic algorithm used for signing or encrypting tokens, such as ES256 or RS256. - | [String] RS256 | 0..1 |
| [Prop] Format | https://www.smart-factory.kr/ids/cd/Format/1/0 Format The format of the security token, such as jwt, cwt, or jwe. - | [String] jwt | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | Oauth2Sc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Oauth2Sc/1/0 |  |  |
| Parent: | Oauth2Sc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Token | https://www.smart-factory.kr/ids/cd/Token/1/0 Token The URI of the token endpoint for obtaining access tokens in OAuth2 flows. - | [String] https://auth.com/token | 0..1 |
| [Prop] Refresh | https://www.smart-factory.kr/ids/cd/Refresh/1/0 Refresh The URI of the endpoint for refreshing expired access tokens using a refresh token. - | [String] https://auth.com/token | 0..1 |
| [Prop] Authorization | https://www.smart-factory.kr/ids/cd/Authorization/1/0 Authorization The URI of the authorization server for obtaining access tokens. - | [String] https://auth.com/token | 0..1 |
| [Prop] Scopes | https://www.smart-factory.kr/ids/cd/Scopes/1/0 Scopes The OAuth2 scopes that define the access permissions requested or granted. - | [String] read | 0..1 |
| [Prop] Flow | https://www.smart-factory.kr/ids/cd/Flow/1/0 Flow The OAuth2 authorization flow type, such as code, client, or device. - | [String] Client | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | InteractionMetadata |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/InteractionMetadata/1/0 |  |  |
| Parent: | InteractionMetadata |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] Properties | https://www.smart-factory.kr/ids/cd/Properties/1/0 Properties A collection of property affordances that expose the readable and writable state of the asset. - | [-] 1 elements | 0..1 |
| [SMC] Actions | https://www.smart-factory.kr/ids/cd/Actions/1/0 Actions A collection of action affordances that can be invoked to perform operations on the asset. - | [-] 0 elements | 0..1 |
| [SMC] Events | https://www.smart-factory.kr/ids/cd/Events/1/0 Events A collection of event affordances that describe asynchronous notifications from the asset. - | [-] 0 elements | 0..1 |

| idShort: | Properties |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Properties/1/0 |  |  |
| Parent: | Properties |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] PropertyName | https://www.smart-factory.kr/ids/cd/PropertyName/1/0 Property name The name identifier of a property within an object schema definition. Current counter value | [-] 13 elements | 1 |

| idShort: | PropertyName |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/PropertyName/1/0 |  |  |
| Parent: | PropertyName |  |  |
| Explanation: | Current counter value@en,Derzeitiger Zählerwert@de,Valore attuale del contatore@it |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Key | https://www.smart-factory.kr/ids/cd/Key/1/0 Key A unique identifier or key used to reference a specific element or value. | [String] temp | 0..1 |
| [Prop] Type | https://www.smart-factory.kr/ids/cd/Type/1/0 Type The data type of a value, such as string, integer, number, boolean, object, or array. | [String] number | 0..1 |
| [Prop] Title | https://www.smart-factory.kr/ids/cd/Title/1/0 Title A human-readable name or title that provides a short description of the interface or element. | [String] temp | 0..1 |
| [Prop] Observable | https://www.smart-factory.kr/ids/cd/Observable/1/0 Observable Indicates whether the property supports observation for value change notifications. | [Boolean] true | 0..1 |
| [Prop] Const | https://www.smart-factory.kr/ids/cd/Const/1/0 Constant A fixed constant value that the property must always have. | [Int] 1 | 0..1 |
| [Prop] Default | https://www.smart-factory.kr/ids/cd/Default/1/0 Default The default value used when no explicit value is provided. | [String] 0 | 0..1 |
| [Prop] Unit | https://www.smart-factory.kr/ids/cd/Unit/1/0 Unit The unit of measurement for the property value, typically expressed as a UN/CEFACT unit code. | [String] degC | 0..1 |
| [Range] MinMax | https://www.smart-factory.kr/ids/cd/min_max/1/0 min max value | [-] 0 100 | 0..1 |
| [Range] LengthRange | https://www.smart-factory.kr/ids/cd/lengthRange/1/0 length range value | [-] 0 100 | 0..1 |
| [SMC] Items | https://www.smart-factory.kr/ids/cd/Items/1/0 Items The schema definition for items contained within an array type property. | [-] 9 elements | 0..1 |
| [Range] ItemsRange | https://www.smart-factory.kr/ids/cd/itemsRange/1/0 items range value | [-] 0 100 | 0..1 |
| [Ref] ValueSemantics | https://www.smart-factory.kr/ids/cd/ValueSemantics/1/0 Value semantics Semantic annotation providing the meaning or context of the property value. | [-] | 0..1 |
| [SMC] Forms | https://www.smart-factory.kr/ids/cd/Forms/1/0 Forms A collection of form definitions that describe how to access or invoke the interaction affordance. | [-] 5 elements | 0..1 |

| idShort: | Items |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Items/1/0 |  |  |
| Parent: | Items |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Type | https://www.smart-factory.kr/ids/cd/Type/1/0 Type The data type of a value, such as string, integer, number, boolean, object, or array. - | [String] number | 0..1 |
| [Prop] Unit | https://www.smart-factory.kr/ids/cd/Unit/1/0 Unit The unit of measurement for the property value, typically expressed as a UN/CEFACT unit code. - | [String] degC | 0..1 |
| [Prop] Default | https://www.smart-factory.kr/ids/cd/Default/1/0 Default The default value used when no explicit value is provided. - | [String] 0 | 0..1 |
| [Prop] Const | https://www.smart-factory.kr/ids/cd/Const/1/0 Constant A fixed constant value that the property must always have. - | [Int] 1 | 0..1 |
| [Prop] Observable | https://www.smart-factory.kr/ids/cd/Observable/1/0 Observable Indicates whether the property supports observation for value change notifications. - | [Boolean] true | 0..1 |
| [Prop] Title | https://www.smart-factory.kr/ids/cd/Title/1/0 Title A human-readable name or title that provides a short description of the interface or element. - | [String] temp | 0..1 |
| [Range] MinMax | https://www.smart-factory.kr/ids/cd/min_max/1/0 min max value | [-] 0 100 | 0..1 |
| [Range] LengthRange | https://www.smart-factory.kr/ids/cd/lengthRange/1/0 length range value | [-] 0 100 | 0..1 |
| [Ref] ValueSemantics | https://admin-shell.io/idta/AssetInterfacesDescription/1/0/valueSemantics - - - | [-] | 0..1 |

| idShort: | Forms |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Forms/1/0 |  |  |
| Parent: | Forms |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Href | https://www.smart-factory.kr/ids/cd/Href/1/0 Hyperlink reference The target URI or relative path for accessing the interaction affordance. - | [String] https://Href.com | 0..1 |
| [Prop] ContentType | https://www.smart-factory.kr/ids/cd/ContentType/1/0 Content type The MIME type indicating the format of the data payload, such as application/json or text/xml. - | [String] text | 0..1 |
| [SML] Security | https://www.smart-factory.kr/ids/cd/Security/1/0 Security Reference to the security configuration that applies to the interface or interaction. - | [-] 1 elements | 0..1 |
| [Prop] HtvMethodName | https://www.smart-factory.kr/ids/cd/HtvMethodName/1/0 HTTP method name The HTTP method to be used for the request, such as GET, POST, PUT, or DELETE. - | [String] GET | 0..1 |
| [SMC] HtvHeaders | https://www.smart-factory.kr/ids/cd/HtvHeaders/1/0 HTTP headers A collection of HTTP header fields to be included in the request. - | [-] 2 elements | 0..1 |

| idShort: | Security |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Security/1/0 |  |  |
| Parent: | Security |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Ref] - | https://www.smart-factory.kr/ids/cd/Security/1/0 Reference to the security configuration that applies to the interface or interaction. | [-] | 1 |

| idShort: | HtvHeaders |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/HtvHeaders/1/0 |  |  |
| Parent: | HtvHeaders |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] HtvFieldName | https://www.smart-factory.kr/ids/cd/HtvFieldName/1/0 HTTP field name The name of an HTTP header field. | [String] Authorization | 1 |
| [Prop] HtvFieldValue | https://www.smart-factory.kr/ids/cd/HtvFieldValue/1/0 HTTP field value The value of an HTTP header field. - | [String] Bearer <access_token> | 1 |

| idShort: | ExternalDescriptor |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/ExternalDescriptor/1/0 |  |  |
| Parent: | ExternalDescriptor |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [File] FileName | https://www.smart-factory.kr/ids/cd/FileName/1/0 File name The name or path of the external descriptor file. - | [-] | 1 |

| idShort: | InterfaceTemplateForMqtt |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/InterfaceTemplateForMqtt/1/0 |  |  |
| Parent: | InterfaceTemplateForMqtt |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Title | https://www.smart-factory.kr/ids/cd/Title/1/0 Title A human-readable name or title that provides a short description of the interface or element. - | [String] Asset Interface Description | 0..1 |
| [Prop] Created | https://www.smart-factory.kr/ids/cd/Created/1/0 Created The date and time when the interface description was initially created. - | [DateTime] 2026-02-01T10:00:00+09:00 | 0..1 |
| [Prop] Modified | https://www.smart-factory.kr/ids/cd/Modified/1/0 Modified The date and time when the interface description was last modified. - | [DateTime] 2026-02-04T09:30:00+09:00 | 0..1 |
| [Prop] Support | https://www.smart-factory.kr/ids/cd/Support/1/0 Support Contact information or URI for obtaining support related to the interface. - | [String] https://www.smart-factory.kr | 0..1 |
| [SMC] EndpointMetadata | https://www.smart-factory.kr/ids/cd/EndpointMetadata/1/0 Endpoint metadata Metadata describing the communication endpoint, including base URI, content type, and security configurations. - | [-] 4 elements | 0..1 |
| [SMC] InteractionMetadata | https://www.smart-factory.kr/ids/cd/InteractionMetadata/1/0 Interaction metadata Metadata describing the available interactions with the asset, including properties, actions, and events. - | [-] 3 elements | 0..1 |
| [SMC] ExternalDescriptor | https://www.smart-factory.kr/ids/cd/ExternalDescriptor/1/0 External descriptor A reference to an external file containing additional interface description, such as a WoT Thing Description. - | [-] 1 elements | 0..1 |

| idShort: | EndpointMetadata |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/EndpointMetadata/1/0 |  |  |
| Parent: | EndpointMetadata |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Base | https://www.smart-factory.kr/ids/cd/Base/1/0 Base The base URI that provides the root address for all relative URIs in the interface. - | [String] https://www.smart-factory.kr | 0..1 |
| [Prop] ContentType | https://www.smart-factory.kr/ids/cd/ContentType/1/0 Content type The MIME type indicating the format of the data payload, such as application/json or text/xml. - | [String] Text/xml | 0..1 |
| [SML] Security | https://www.smart-factory.kr/ids/cd/Security/1/0 Security Reference to the security configuration that applies to the interface or interaction. - | [-] 1 elements | 0..1 |
| [SMC] SecurityDefinitions | https://www.smart-factory.kr/ids/cd/SecurityDefinitions/1/0 Security definitions A collection of security scheme definitions that can be referenced by security configurations. - | [-] 9 elements | 0..1 |

| idShort: | Security |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Security/1/0 |  |  |
| Parent: | Security |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Ref] - | https://www.smart-factory.kr/ids/cd/Security/1/0 Reference to the security configuration that applies to the interface or interaction. | [-] | 1 |

| idShort: | SecurityDefinitions |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/SecurityDefinitions/1/0 |  |  |
| Parent: | SecurityDefinitions |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] NosecSc | https://www.smart-factory.kr/ids/cd/NosecSc/1/0 No security scheme A security scheme indicating that no authentication or security mechanism is required. - | [-] 1 elements | 0..1 |
| [SMC] AutoSc | https://www.smart-factory.kr/ids/cd/AutoSc/1/0 Auto security scheme A security scheme where the security mechanism is automatically determined by the protocol or context. - | [-] 2 elements | 0..1 |
| [SMC] BasicSc | https://www.smart-factory.kr/ids/cd/BasicSc/1/0 Basic security scheme A security scheme using HTTP Basic Authentication with username and password credentials. - | [-] 4 elements | 0..1 |
| [SMC] ComboSc | https://www.smart-factory.kr/ids/cd/ComboSc/1/0 Combo security scheme A security scheme that combines multiple security schemes using logical AND or OR operations. - | [-] 4 elements | 0..1 |
| [SMC] ApikeySc | https://www.smart-factory.kr/ids/cd/ApikeySc/1/0 API key security scheme A security scheme using an API key for authentication, typically passed in a header or query parameter. - | [-] 4 elements | 0..1 |
| [SMC] PskSc | https://www.smart-factory.kr/ids/cd/PskSc/1/0 Pre-shared key security scheme A security scheme using a pre-shared key (PSK) for authentication, commonly used in CoAP and DTLS. - | [-] 3 elements | 0..1 |
| [SMC] DigestSc | https://www.smart-factory.kr/ids/cd/DigestSc/1/0 Digest security scheme A security scheme using HTTP Digest Authentication with challenge-response mechanism. - | [-] 5 elements | 0..1 |
| [SMC] BearerSc | https://www.smart-factory.kr/ids/cd/BearerSc/1/0 Bearer security scheme A security scheme using bearer tokens, typically JWT or OAuth2 access tokens. - | [-] 7 elements | 0..1 |
| [SMC] Oauth2Sc | https://www.smart-factory.kr/ids/cd/Oauth2Sc/1/0 OAuth2 security scheme A security scheme using OAuth 2.0 protocol for authorization and access control. - | [-] 7 elements | 0..1 |

| idShort: | NosecSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/NosecSc/1/0 |  |  |
| Parent: | NosecSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] apikey | 1 |

| idShort: | AutoSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/AutoSc/1/0 |  |  |
| Parent: | AutoSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] Oauth2 | 1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 1 |

| idShort: | BasicSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/BasicSc/1/0 |  |  |
| Parent: | BasicSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Name | https://www.smart-factory.kr/ids/cd/Name/1/0 Name Human-readable name of the sensor. - | [String] Authorization | 0..1 |
| [Prop] In | https://www.smart-factory.kr/ids/cd/In/1/0 In The location where the security credential is expected, such as header, query, body, or cookie. - | [String] header | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | ComboSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/ComboSc/1/0 |  |  |
| Parent: | ComboSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [SML] OneOf | https://www.smart-factory.kr/ids/cd/OneOf/1/0 One of A list of security schemes where at least one must be satisfied (logical OR combination). - | [-] 0 elements | 0..1 |
| [SML] AllOf | https://www.smart-factory.kr/ids/cd/AllOf/1/0 All of A list of security schemes where all must be satisfied (logical AND combination). - | [-] 0 elements | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | ApikeySc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/ApikeySc/1/0 |  |  |
| Parent: | ApikeySc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Name | https://www.smart-factory.kr/ids/cd/Name/1/0 Name Human-readable name of the sensor. - | [String] Authorization | 0..1 |
| [Prop] In | https://www.smart-factory.kr/ids/cd/In/1/0 In The location where the security credential is expected, such as header, query, body, or cookie. - | [String] header | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | PskSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/PskSc/1/0 |  |  |
| Parent: | PskSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Identity | https://www.smart-factory.kr/ids/cd/Identity/1/0 Identity The identity hint used to identify the pre-shared key in PSK security schemes. - | [String] examplekey | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | DigestSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/DigestSc/1/0 |  |  |
| Parent: | DigestSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Name | https://www.smart-factory.kr/ids/cd/Name/1/0 Name Human-readable name of the sensor. - | [String] Authorization | 0..1 |
| [Prop] In | https://www.smart-factory.kr/ids/cd/In/1/0 In The location where the security credential is expected, such as header, query, body, or cookie. - | [String] header | 0..1 |
| [Prop] Qop | https://www.smart-factory.kr/ids/cd/Qop/1/0 Quality of protection The quality of protection value for digest authentication, such as auth or auth-int. - | [String] auth | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | BearerSc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/BearerSc/1/0 |  |  |
| Parent: | BearerSc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Name | https://www.smart-factory.kr/ids/cd/Name/1/0 Name Human-readable name of the sensor. - | [String] Authorization | 0..1 |
| [Prop] In | https://www.smart-factory.kr/ids/cd/In/1/0 In The location where the security credential is expected, such as header, query, body, or cookie. - | [String] header | 0..1 |
| [Prop] Authorization | https://www.smart-factory.kr/ids/cd/Authorization/1/0 Authorization The URI of the authorization server for obtaining access tokens. - | [String] https://auth.com/token | 0..1 |
| [Prop] Alg | https://www.smart-factory.kr/ids/cd/Alg/1/0 Algorithm The cryptographic algorithm used for signing or encrypting tokens, such as ES256 or RS256. - | [String] RS256 | 0..1 |
| [Prop] Format | https://www.smart-factory.kr/ids/cd/Format/1/0 Format The format of the security token, such as jwt, cwt, or jwe. - | [String] jwt | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | Oauth2Sc |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Oauth2Sc/1/0 |  |  |
| Parent: | Oauth2Sc |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Scheme | https://www.smart-factory.kr/ids/cd/Scheme/1/0 Scheme The type identifier of the security scheme, such as basic, bearer, apikey, or oauth2. - | [String] bearer | 0..1 |
| [Prop] Token | https://www.smart-factory.kr/ids/cd/Token/1/0 Token The URI of the token endpoint for obtaining access tokens in OAuth2 flows. - | [String] https://auth.com/token | 0..1 |
| [Prop] Refresh | https://www.smart-factory.kr/ids/cd/Refresh/1/0 Refresh The URI of the endpoint for refreshing expired access tokens using a refresh token. - | [String] https://auth.com/token | 0..1 |
| [Prop] Authorization | https://www.smart-factory.kr/ids/cd/Authorization/1/0 Authorization The URI of the authorization server for obtaining access tokens. - | [String] https://auth.com/token | 0..1 |
| [Prop] Scopes | https://www.smart-factory.kr/ids/cd/Scopes/1/0 Scopes The OAuth2 scopes that define the access permissions requested or granted. - | [String] read | 0..1 |
| [Prop] Flow | https://www.smart-factory.kr/ids/cd/Flow/1/0 Flow The OAuth2 authorization flow type, such as code, client, or device. - | [String] Client | 0..1 |
| [Prop] Proxy | https://www.smart-factory.kr/ids/cd/Proxy/1/0 Proxy The URI of a proxy server to be used for accessing the security mechanism or endpoint. - | [String] http://proxy.com:8080 | 0..1 |

| idShort: | InteractionMetadata |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/InteractionMetadata/1/0 |  |  |
| Parent: | InteractionMetadata |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] Properties | https://www.smart-factory.kr/ids/cd/Properties/1/0 Properties A collection of property affordances that expose the readable and writable state of the asset. - | [-] 1 elements | 0..1 |
| [SMC] Actions | https://www.smart-factory.kr/ids/cd/Actions/1/0 Actions A collection of action affordances that can be invoked to perform operations on the asset. - | [-] 0 elements | 0..1 |
| [SMC] Events | https://www.smart-factory.kr/ids/cd/Events/1/0 Events A collection of event affordances that describe asynchronous notifications from the asset. - | [-] 0 elements | 0..1 |

| idShort: | Properties |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Properties/1/0 |  |  |
| Parent: | Properties |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] PropertyName | https://www.smart-factory.kr/ids/cd/PropertyName/1/0 Property name The name identifier of a property within an object schema definition. Current counter value | [-] 13 elements | 1 |

| idShort: | PropertyName |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/PropertyName/1/0 |  |  |
| Parent: | PropertyName |  |  |
| Explanation: | Current counter value@en,Derzeitiger Zählerwert@de,Valore attuale del contatore@it |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Key | https://www.smart-factory.kr/ids/cd/Key/1/0 Key A unique identifier or key used to reference a specific element or value. | [String] temp | 0..1 |
| [Prop] Type | https://www.smart-factory.kr/ids/cd/Type/1/0 Type The data type of a value, such as string, integer, number, boolean, object, or array. | [String] number | 0..1 |
| [Prop] Title | https://www.smart-factory.kr/ids/cd/Title/1/0 Title A human-readable name or title that provides a short description of the interface or element. | [String] temp | 0..1 |
| [Prop] Observable | https://www.smart-factory.kr/ids/cd/Observable/1/0 Observable Indicates whether the property supports observation for value change notifications. | [Boolean] true | 0..1 |
| [Prop] Const | https://www.smart-factory.kr/ids/cd/Const/1/0 Constant A fixed constant value that the property must always have. | [Int] 1 | 0..1 |
| [Prop] Default | https://www.smart-factory.kr/ids/cd/Default/1/0 Default The default value used when no explicit value is provided. | [String] 0 | 0..1 |
| [Prop] Unit | https://www.smart-factory.kr/ids/cd/Unit/1/0 Unit The unit of measurement for the property value, typically expressed as a UN/CEFACT unit code. | [String] degC | 0..1 |
| [Range] MinMax | https://www.smart-factory.kr/ids/cd/min_max/1/0 min max value | [-] 0 100 | 0..1 |
| [Range] LengthRange | https://www.smart-factory.kr/ids/cd/lengthRange/1/0 length range value | [-] 0 100 | 0..1 |
| [SMC] Items | https://www.smart-factory.kr/ids/cd/Items/1/0 Items The schema definition for items contained within an array type property. | [-] 9 elements | 0..1 |
| [Range] ItemsRange | https://www.smart-factory.kr/ids/cd/itemsRange/1/0 items range value | [-] 0 100 | 0..1 |
| [Ref] ValueSemantics | https://www.smart-factory.kr/ids/cd/ValueSemantics/1/0 Value semantics Semantic annotation providing the meaning or context of the property value. | [-] | 0..1 |
| [SMC] Forms | https://www.smart-factory.kr/ids/cd/Forms/1/0 Forms A collection of form definitions that describe how to access or invoke the interaction affordance. | [-] 5 elements | 0..1 |

| idShort: | Items |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Items/1/0 |  |  |
| Parent: | Items |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Type | https://www.smart-factory.kr/ids/cd/Type/1/0 Type The data type of a value, such as string, integer, number, boolean, object, or array. - | [String] number | 0..1 |
| [Prop] Unit | https://www.smart-factory.kr/ids/cd/Unit/1/0 Unit The unit of measurement for the property value, typically expressed as a UN/CEFACT unit code. - | [String] degC | 0..1 |
| [Prop] Default | https://www.smart-factory.kr/ids/cd/Default/1/0 Default The default value used when no explicit value is provided. - | [String] 0 | 0..1 |
| [Prop] Const | https://www.smart-factory.kr/ids/cd/Const/1/0 Constant A fixed constant value that the property must always have. - | [Int] 1 | 0..1 |
| [Prop] Observable | https://www.smart-factory.kr/ids/cd/Observable/1/0 Observable Indicates whether the property supports observation for value change notifications. - | [Boolean] true | 0..1 |
| [Prop] Title | https://www.smart-factory.kr/ids/cd/Title/1/0 Title A human-readable name or title that provides a short description of the interface or element. - | [String] temp | 0..1 |
| [Range] MinMax | https://www.smart-factory.kr/ids/cd/min_max/1/0 min max value | [-] 0 100 | 0..1 |
| [Range] LengthRange | https://www.smart-factory.kr/ids/cd/lengthRange/1/0 length range value | [-] 0 100 | 0..1 |
| [Ref] ValueSemantics | https://admin-shell.io/idta/AssetInterfacesDescription/1/0/valueSemantics - - - | [-] | 0..1 |

| idShort: | Forms |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Forms/1/0 |  |  |
| Parent: | Forms |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] Href | https://www.smart-factory.kr/ids/cd/Href/1/0 Hyperlink reference The target URI or relative path for accessing the interaction affordance. - | [String] https://Href.com | 0..1 |
| [Prop] ContentType | https://www.smart-factory.kr/ids/cd/ContentType/1/0 Content type The MIME type indicating the format of the data payload, such as application/json or text/xml. - | [String] text | 0..1 |
| [SML] Security | https://www.smart-factory.kr/ids/cd/Security/1/0 Security Reference to the security configuration that applies to the interface or interaction. - | [-] 1 elements | 0..1 |
| [Prop] HtvMethodName | https://www.smart-factory.kr/ids/cd/HtvMethodName/1/0 HTTP method name The HTTP method to be used for the request, such as GET, POST, PUT, or DELETE. - | [String] GET | 0..1 |
| [SMC] HtvHeaders | https://www.smart-factory.kr/ids/cd/HtvHeaders/1/0 HTTP headers A collection of HTTP header fields to be included in the request. - | [-] 2 elements | 0..1 |

| idShort: | Security |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementList |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/Security/1/0 |  |  |
| Parent: | Security |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Ref] - | https://www.smart-factory.kr/ids/cd/Security/1/0 Reference to the security configuration that applies to the interface or interaction. | [-] | 1 |

| idShort: | HtvHeaders |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/HtvHeaders/1/0 |  |  |
| Parent: | HtvHeaders |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] HtvFieldName | https://www.smart-factory.kr/ids/cd/HtvFieldName/1/0 HTTP field name The name of an HTTP header field. | [String] Authorization | 1 |
| [Prop] HtvFieldValue | https://www.smart-factory.kr/ids/cd/HtvFieldValue/1/0 HTTP field value The value of an HTTP header field. - | [String] Bearer <access_token> | 1 |

| idShort: | ExternalDescriptor |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/ExternalDescriptor/1/0 |  |  |
| Parent: | ExternalDescriptor |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [File] FileName | https://www.smart-factory.kr/ids/cd/FileName/1/0 File name The name or path of the external descriptor file. - | [-] | 1 |

자. Hierarchical Structures submodel의 하위 Property는 다음과 같이 구성된다.

| idShort: | HierarchicalStructures |  |  |
| --- | --- | --- | --- |
| Class: | Submodel |  |  |
| semanticId: | https://admin-shell.io/idta/HierarchicalStructures/1/1/Submodel |  |  |
| Parent: | HierarchicalStructures |  |  |
| Explanation: | The Submodel HierarchicalStructures identified by its semanticId. The Submodel idShort can be picked freely.@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Ent] PartRemovalRobot | https://admin-shell.io/idta/HierarchicalStructures/EntryNode/1/0 Base entry point for the Entity tree in this Submodel, this must be a Self-managed Entity reflecting the Assets administrated in the AAS this Submodel is part of. Base entry point for the Entity tree in this Submodel, this must be a Self-managed Entity reflecting the Assets administrated in the AAS this Submodel is part of. | [-] - |  |
| [Prop] ArcheType | https://admin-shell.io/idta/HierarchicalStructures/ArcheType/1/0 ArcheType of the Submodel, there are three allowed enumeration entries: 1. “Full”, 2. “OneDown” and 3. “OneUp”. These entries reflect the structure of the Submodel as defined in 1.5.1.3 & 1.5.1.4. ArcheType of the Submodel, there are three allowed enumeration entries: 1. “Full”, 2. “OneDown” and 3. “OneUp”. | [String] |  |

| idShort: | PartRemovalRobot |  |  |
| --- | --- | --- | --- |
| Class: | Entity |  |  |
| semanticId: | https://admin-shell.io/idta/HierarchicalStructures/EntryNode/1/0 |  |  |
| Parent: | PartRemovalRobot |  |  |
| Explanation: | Base entry point for the Entity tree in this Submodel, this must be a Self-managed Entity reflecting the Assets administrated in the AAS this Submodel is part of.@en |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Ent] MainUnit | https://admin-shell.io/idta/HierarchicalStructures/Node/1/0 The Entity Node can be a co-managed or self-managed entity representing an asset in the hierarchical structure. Note: The idShort can be chosen freely. At least one nested Node shall be created as a SubmodelElement for the EntryNode. In relation to the ArcheType, either the Relationship “IsPartOf” or “HasPart” shall be created using this Node as Second attribute. Base entry point for the Entity tree in this Submodel, this must be a Self-managed Entity reflecting the Assets administrated in the Asset Administration Shell this Submodel is part of. The idShort of the EntryNode can be picked freely and may reflect a name of the asset. | [-] - |  |
| [Rel] HasPart_MainUnit | https://admin-shell.io/idta/HierarchicalStructures/HasPart/1/0 Modelling of logical connections between components and sub-components. Either this or "IsPartOf" must be used, not both. “First” and “Second” attributes must contain either a EntryNode or a Node. The relationships shall only reference EntryNodes or Nodes in the same Submodel. Note: The idShort can be chosen freely - | [-] |  |
| [Ent] GripperUnit | https://admin-shell.io/idta/HierarchicalStructures/Node/1/0 The Entity Node can be a co-managed or self-managed entity representing an asset in the hierarchical structure. Note: The idShort can be chosen freely. At least one nested Node shall be created as a SubmodelElement for the EntryNode. In relation to the ArcheType, either the Relationship “IsPartOf” or “HasPart” shall be created using this Node as Second attribute. Base entry point for the Entity tree in this Submodel, this must be a Self-managed Entity reflecting the Assets administrated in the Asset Administration Shell this Submodel is part of. The idShort of the EntryNode can be picked freely and may reflect a name of the asset. | [-] - |  |
| [Rel] HasPart_GripperUnit | https://admin-shell.io/idta/HierarchicalStructures/HasPart/1/0 Modelling of logical connections between components and sub-components. Either this or "IsPartOf" must be used, not both. “First” and “Second” attributes must contain either a EntryNode or a Node. The relationships shall only reference EntryNodes or Nodes in the same Submodel. Note: The idShort can be chosen freely - | [-] [ |  |

차. Gripper Management submodel의 하위 Property는 다음과 같이 구성된다.

| idShort: | GripperManagement |  |  |
| --- | --- | --- | --- |
| Class: | Submodel |  |  |
| semanticId: | https://www.smart-factory.kr/ids/sm/PartRemovalRobot/GripperManagement/1/0 |  |  |
| Parent: | GripperManagement |  |  |
| Explanation: | - |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [SMC] GripperSpecification | https://www.smart-factory.kr/ids/cd/GripperSpecification/1/0 Gripper specification Specification data of the gripper used for green body handling. Gripper specification | [-] 4 elements | - |
| [SMC] GripperMaintenanceHistory | https://www.smart-factory.kr/ids/cd/GripperMaintenanceHistory/1/0 Gripper maintenance history Maintenance history data for the gripper, including dates, actions, and replaced parts. Maintenance history of the gripper | [-] 4 elements | - |

| idShort: | GripperSpecification |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/GripperSpecification/1/0 |  |  |
| Parent: | GripperSpecification |  |  |
| Explanation: | Gripper specification@en,그리퍼 사양@kr |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] GripperType | https://www.smart-factory.kr/ids/cd/GripperType/1/0 Gripper type Type of gripper mechanism used to pick and place the green body. Type of gripper (e.g., vacuum, mechanical, magnetic) | [String] Vacuum Grippper | - |
| [Prop] FingerMaterial | https://www.smart-factory.kr/ids/cd/FingerMaterial/1/0 Finger material Material of the gripper finger or contact surface that touches the green body. Material of the gripper finger or contact surface | [String] Stainless Steel | - |
| [Prop] MaxPayload | https://www.smart-factory.kr/ids/cd/MaxPayload/1/0 Maximum payload Maximum load mass that the gripper can safely handle. Maximum payload that the gripper can handle | [Double] 2.5 | - |
| [Prop] MaxOpeningWidth | https://www.smart-factory.kr/ids/cd/MaxOpeningWidth/1/0 Maximum opening width Maximum opening width of the gripper jaws or fingers. Maximum opening width of the gripper | [Double] 120.0 | - |

| idShort: | GripperMaintenanceHistory |  |  |
| --- | --- | --- | --- |
| Class: | SubmodelElementCollection |  |  |
| semanticId: | https://www.smart-factory.kr/ids/cd/GripperMaintenanceHistory/1/0 |  |  |
| Parent: | GripperMaintenanceHistory |  |  |
| Explanation: | Maintenance history of the gripper@en,그리퍼 유지보수 이력@kr |  |  |
| [SME type] idShort | semanticId Description@en | [valueType] example | card. |
| [Prop] LastMaintenanceDate | https://www.smart-factory.kr/ids/cd/LastMaintenanceDate/1/0 Last maintenance date Date when the most recent maintenance activity was performed on the gripper. Date of last maintenance activity | [Date] 2026-01-01 | - |
| [Prop] MaintenanceAction | https://www.smart-factory.kr/ids/cd/MaintenanceAction/1/0 Maintenance action Description of the maintenance action performed for the gripper. Description of maintenance action performed | [String] Vacuum pad cleaning and suction test | - |
| [Prop] ReplacedPart | https://www.smart-factory.kr/ids/cd/ReplacedPart/1/0 Replaced part Name or identifier of a part replaced during gripper maintenance. Replaced part name or identifier | [String] V-PAD-05-SILICONE | - |
| [Prop] NextMaintenanceDue | https://www.smart-factory.kr/ids/cd/NextMaintenanceDue/1/0 Next maintenance due Planned date when the next maintenance activity is due for the gripper. Planned next maintenance date | [Date] 2026-01-01 | - |

IV.AAS 참조모델 활용 방법

1. 참조모델 활용

○ 참조모델을 활용할 때에는 아래의 사항들을 참조하는 것을 권장한다.
- 참조모델의 submodel은 가급적 수정하지 않고, 필요에 따라 별도의 submodel을 추가한다.
- 필요한 property를 추가하고 해당 semantic 정보를 입력한다.

2. 신규 AAS 작성 및 수정

가. AASX PackageExplorer 다운로드

○ AAS를 작성하기 위한 일반적인 방법으로는 AASX PackageExplorer를 사용하는 것이 일반적이다.

○ AASX PackageExplorer는 아래의 링크를 통해 아래 그림의 프로그램 중 본인의 목적에 맞는 파일을 다운받아 압축을 푼 뒤 AasxPackageExplorer.exe 파일을 실행하면 아래 두 번째 그림과 같이 AASX PackageExplorer의 기본 화면이 나타난다.
https://github.com/admin-shell-io/aasx-package-explorer/releases

그림 16 AASX PackageExplorer 다운로드 경로

그림 17 AASX Package Explorer 파일 실행 초기화면

나. Edit 모드

○ AAS 생성, Submodel 생성 등의 기능을 수행하기 위해서 아래 첫 번째 그림과 같이 ‘Eidt’모드를 활성화시킨다. 이를 위해 아래 두 번째 그림과 같이, 상단의 작업창에서 [Workspace] - [Edit]모드를 클릭한다.

그림 18 활성화된 Edit 모드

그림 19 Edit 모드 활성화 단계

다. AAS 생성

○ AAS를 생성하기 위해 앞서 Edit모드를 활성화시킨 상단의 Edit 모드 화면을 기준으로, [AdministrationShells] - [Add AAS]를 클릭하면 아래 첫 번째 AAS 생성 그림과 같은 화면이 나타난다. 아래 두 번째 AAS 정보 그림에서 오른쪽에 ‘Element’에서는 AAS 정보를 입력할 수 있다.

그림 20 AAS 생성

그림 21 AAS 정보

라. SubmodelTemplate을 이용한 Submodel 추가

○ 먼저, File-Open Auxiliary AAS 버튼을 통해 IDTA의 Nameplate Submodel을 추가하는 것을 예시로 설명한다. 아래 첫 번째 그림의 IDTA 홈페이지(https://industrialdigitaltwin.org/en)의 Content Hub에 게시된 SubmodelTemplate을 찾는다. 아래 두 번째 그림과 같이 찾은 SubmodelTemplate의 깃허브 주소로 이동하여, AASX 파일을 다운받아 File - Open Auxiliary AAS 버튼을 누르고, 다운받은 AASX 파일을 선택한다.

그림 22 IDTA SubmodelTemplate 다운로드

그림 23 IDTA SubmodelTemplate 파일 열기

○ 아래 그림의 상단 화면과 같이 AAS를 눌러 우측 Element-Copy from existing Submodel-Copy recursive를 누른다. 새로 표시된 창에서 추가한 AAS 모델 하위에 있는 Submodel을 선택한 후 Select 버튼을 눌러 불러온다. 아래 그림의 하단 화면의 좌측에서 불러온 Submodel을 누른 뒤 우측 Element 영역에서 Property를 추가하거나 값을 수정할 수 있다. 하위 Property를 선택하면 일부 값만 선택하여 변경할 수 있다. 변경 확인 버튼은 Take over changes 버튼을 클릭하여 반영을 완료한다.

그림 24 Template 이용 Submodel 생성

○ 아래 그림은 같은 방법으로 추가한 3종(TechnicalData, HierarchicalStructures, DigitalNameplate)의 서브모델 리스트이다.

그림 25 동일한 방법으로 추가한 3종의 Submodel

마. Submodel 및 SubmodelCollection 생성

○ 참조모델에 정의되지 않은 내용은 개별적으로 작성하여야 한다. 개별 Submodel 및 SubmodelCollection을 생성하기 위해 아래 그림과 같이 이전에 생성된 AAS를 클릭한 후, 우측의 ‘Element’에서 [Create new Submodel of kind Template] 또는 [Create new Submodel of kind Instance]를 클릭한다.

그림 26 Submodel 생성

○ 생성된 Submodel을 클릭한 뒤 아래 그림과 같이 우측의 ‘Element’에서 [Add collection]을 클릭하여 SubmodelCollection을 생성한다.

그림 27 SubmodelCollection 생성

○ 아래 그림의 오른쪽 ‘Element’에서는 SubmodelCollection 정보를 입력할 수 있다.

그림 28 SubmodelCollection 정보

바. Property 생성

○ 아래 그림과 같이 Submodel 또는 SubmodelCollection을 클릭한 뒤, 우측의 ‘Element’에서 [Add Property]를 클릭하여 신규 Property를 생성한다.

그림 29 Property 생성

3. AAS 모델(템플릿)을 실제 제품별 AAS 인스턴스로 전환

가. AssetInformation 수정

○ Edit 모드에서 아래 그림과 같이 AssetInformation을 선택 시 표시되는 오른쪽 Element 메뉴의 kind 값을 Instance로 선택한다.

그림 30 AssetInformation의 kind 변경

○ 실제 제품의 고유한 global id(IRI 형태)를 부여하여, 이 값을 아래의 그림과 같이 AssetInformation의 globalAssetId 부분에 입력한다.

일반적으로 자산(Asset)이 인스턴스화 되면, 개별 인스턴스마다 고유한 식별정보가 부여된다. 이는 제품의 유형 및 제품 생산 기업의 내부 지침에 따라 다양한 형식으로 부여될 것이며, Asset Information 의 Global Asset Id에는 개별 제품을 구분하고 식별할 수 있는 이러한 고유 식별정보가 부여되어야 한다. 또한, 모든 Asset Administration Shell은 자산이 생산된 시점부터, 자산에 부여된 global asset id를 사용하여 대상 자산을 구분하므로, 이 식별자는 제품의 폐기시까지 변경 없이 유지되어야 한다.

그림 31 AssetInformation의 globalAssetId 입력

○ 마지막으로, 아래 그림과 같이 AssetInformation의 assetType을 Instance로 입력한다.

그림 32 AssetInformation의 assetType 입력

나. Submodel수정

○ Edit 모드에서 아래 그림의 서브모델의 Kind (of model) 메뉴의 Instance를 선택한다.

그림 33 Submodel 인스턴스

다. Property 수정

○ 서브모델에 있는 Property중에서, 개별 제품마다 변경되는 값들을 입력한다. 예를 들어 SerialNumber / DateOfManufacture / FirmwareVersion등과 같은 제품별 값들을 입력한다.

그림 34 Property 인스턴스

라. AAS 모델을 인스턴스 변환 결과

○ 아래 그림은 인스턴스 변환 결과를 비교한 화면이다. AssetInformation과 Indentificatoin 서브모델에서의 인스턴스 전과 후의 결과이다. 개별 제품별로 달라지는 이러한 프로퍼티들은, 모델에 적용된 서브모델의 종류나 프로퍼티의 구성에 따라 달라질 수 있으며, 이 가이드 문서에서는 가장 대표적인 프로퍼티를 대상으로 예시 value를 기술하였다.

그림 35 AAS 인스턴스 변환 전 화면

그림 36 AAS 인스턴스 변환 후 화면

4. 분말 성형 공정 AAS 참조모델 활용 시나리오

가. 개요

○ 분말성형장치 AAS는 장비의 전 생애주기(설계, 제조, 운영, 유지보수) 동안 발생하는 데이터를 표준화된 구조로 관리합니다. 이를 통해 제조 현장의 지능화, 자동화 및 자율제조 구현을 위한 기반을 제공합니다.

○ 본 시나리오는 분말성형 공정 내 주요 장비 4종(혼련기, 성형기, 로봇, 혼련기)을 AAS 기반으로 디지털화하여, 장비 간 데이터 표준화와 상호운용성을 확보하는 데 목적이 있습니다. 이를 통해 공정 전반의 추적성(Traceability), 효율성(Efficiency), 유연성(Flexibility)을 극대화합니다.

그림 38 분말 성형 공정 AAS 참조모델 활용 시나리오

나. 시나리오 1: 자산 식별 및 디지털 트윈 온보딩

○ 목적: 공정 내 장비의 사양 정보를 표준화된 형태로 관리하고 시스템에 자동 등록

○ 활용 Submodel 또는 SMC: Identification, TechnicalData

○ 주요 프로퍼티(Property) 활용:

- 혼련기: ManufacturerName, MixingCapacity, MaxRPM

- 혼련기: MaxTemperature, HeaterType, ChamberSize

○ 세부 내용:

- 장비 도입 시 AAS 파일을 상위 관제 시스템(CAMS)에 업로드하여 수동 입력 없이 장비 제원을 자동 등록하고, DigitalNameplate 서브모델을 통해 인증서 및 매뉴얼을 즉각 참조

그림 39 자산 식별 및 디지털 트윈 온보딩

다. 시나리오 2: 공정 실시간 모니터링 및 이상 탐지

○ 목적: 가동 중 발생하는 실시간 데이터를 수집하여 공정 안정성을 확보

○ 활용 Submodel 또는 SMC: OperationalData

○ 주요 프로퍼티(Property) 활용:

- 분말성형장치: PressingForce (kN), StrokePosition (mm), CycleTime

- 혼련기: ViscosityValue, TorqueValue, MotorTemperature

○ 세부 내용:

- AAS의 OperationalData 내 DynamicProperty를 통해 초당 데이터를 수집. 성형 압력이 설정된 임계값(Threshold)을 벗어날 경우 알람을 발생시키고 AAS의 ContactInformation에 등록된 담당자에게 즉시 통보

그림 40 공정 실시간 모니터링 및 이상 탐지

라. 시나리오 3: 소결 공정 에너지 효율 최적화

○ 목적: 전력 소비가 가장 큰 혼련기의 에너지 사용량을 분석하여 탄소 중립 및 비용 절감 달성

○ 활용 Submodel 또는 SMC: OperationalData/EnergyEfficiency, OperationalData/Energy Monitoring

○ 주요 프로퍼티(Property) 활용:

- 혼련기: PowerConsumption (kWh), GasFlowRate, IdlePower

○ 세부 내용:

- 혼련기의 AAS 내 EnergyEfficiency 서브모델을 통해 대기 시간 및 가열 단계별 에너지 소비량을 측정. 생산량 대비 에너지 효율(EEI)을 계산하여 최적의 적재량(Batch Size) 제안

그림 41 소결 공정 에너지 효율 최적화

마. 시나리오 4: 생산 이력 기반 품질 역추적 (Quality Traceability)

○ 목적: 완제품 불량 발생 시, 혼련부터 소결까지의 전 공정 데이터를 연계 분석

○ 활용 Submodel 또는 SMC: ProcessInformation, History, MaterialManagement, Operational Data (Process Control, Production Monitoring)

○ 주요 프로퍼티(Property) 활용:

- 전 공정 공통: BatchID, ProductID, Timestamp.

- 혼련기: OxygenLevel, HeatingProfile (온도 곡선)

○ 세부 내용:

- 특정 BatchID의 제품에서 크랙 발생 시, 해당 ID가 포함된 혼련기의 Torque 이력과 혼련기의 OxygenLevel 데이터를 AAS에서 통합 추출. 상관관계 분석을 통해 "혼련 불충분" 또는 "산화 반응" 등의 원인을 즉각 파악

그림 42 생산 이력 기반 품질 역추적

바. 시나리오 5: 데이터 기반 예지보전 (Predictive Maintenance)

○ 목적: 취출로봇 및 성형기의 핵심 부품 고장을 사전 예측하여 가동 중단 최소화

○ 활용 Submodel 또는 SMC: ConditionMonitoring, Maintenance

○ 주요 프로퍼티(Property) 활용:

- 취출로봇: JointVibration (Hz), MotorCurrent, CumulativeDistance

- 분말성형장치: OilPressureStatus, SealWearLevel

○ 세부 내용:

- AAS의 ConditionMonitoring 서브모델에 실시간 진동 데이터를 누적. AI 모델이 Joint Vibration 프로퍼티의 변화 패턴을 감지하여 고장 징후를 발견하면, Maintenance 서브모델의 NextMaintenance Date를 자동으로 갱신하고 예비 부품 재고 확인

그림 43 데이터 기반 예지보전

사. 시나리오 6: 장비 간 협동 제어 및 자율 구성 (Plug & Produce)

○ 목적: 성형 장치와 로봇 간의 인터페이스를 AAS를 통해 표준화하여 유연한 공정 구성

○ 활용 Submodel 또는 SMC: Capability, Interface

○ 주요 프로퍼티(Property) 활용:

- 취출로봇: PayloadCapacity, WorkingRadius, GripperState

- 분말성형장치: EjectionSignal, MoldOpenStatus

○ 세부 내용:

- 성형기가 교체되더라도 AAS의 Capability 서브모델(예: '제품 취출 가능 신호 전송')을 통해 로봇이 새로운 장비의 통신 접점을 자동으로 인식. 별도의 PLC 프로그래밍 없이 장비 간 Interface 프로퍼티 매핑만으로 동기화된 협업 수행

그림 44 장비 간 협동 제어 및 자율 구성

아. 시나리오 7: 원재료 기반의 지능형 공정 제어 및 품질 추적 관리

○ 분말의 물리적 특성 데이터와 실시간 공정 데이터를 연계하여 제품의 품질을 일관되게 유지하는 시나리오입니다.

○ 활용 Submodel 또는 SMC: MaterialManagement, OperationalData (Process Control, Production Monitoring).

○ 세부 내용:

- 분말 물성 기반 조건 최적화: MaterialManagement에서 관리되는 분말의 겉보기 밀도(ISO 3923-1) 및 유동도(ASTM B213) 데이터를 기반으로 OperationalData의 충진 깊이(Filling Depth)나 압축 압력(Compression Force) 파라미터를 자동으로 최적화합니다.

- 품질 문제 원인 분석: 불량 발생 시 MaterialManagement의 로트(Lot) 번호와 OperationalData의 실시간 공정 수치(압력, 온도 등)를 연계 분석하여 원재료 결함인지 공정 변수 문제인지 신속하게 파악합니다.

- 실시간 품질 모니터링: ProductionMonitoring의 카운터 정보와 MaterialHopperLevel을 실시간으로 비교하여 원재료 소비 패턴과 생산 효율을 관리합니다.

그림 45 원재료 기반의 지능형 공정 제어 및 품질 추적

자. 시나리오 8: 예지 보전 및 에너지 효율 관리

○ 장비 상태의 실시간 모니터링을 통해 고장을 예방하고 에너지 소비를 최적화하는 시나리오입니다.

○ 활용 Submodel 또는 SMC: OperationalData (Maintenance Monitoring, Energy Monitoring), Hierarchical Structures.

○ 세부 내용:

- 핵심 부품별 상태 감시: HierarchicalStructures로 정의된 유압 펌프, 실린더 등 하위 자산별로 MaintenanceMonitoring의 온도 및 알람 데이터를 수집하여 이상 징후를 조기에 발견합니다.

- 예측 유지보수: 모터 온도(MotorTemperature), 유압유 온도(HydraulicOilTemp) 등의 트렌드를 분석하여 고장이 발생하기 전에 정비 시점을 결정함으로써 다운타임을 최소화합니다.

- 공정 에너지 최적화: EnergyMonitoring을 통해 수집된 실시간 전력 소비량과 역률 데이터를 분석하여 대기 전력을 절감하거나 공정별 에너지 소비 효율을 개선합니다.

그림 46 예지 보전 및 에너지 효율 관리

차. 시나리오 9: 장비 라이프사이클 및 구성 변경 관리

○ 장비의 개조, 수리 및 부품 교체 이력을 디지털 형상 정보와 함께 관리하는 시나리오입니다.

○ 활용 Submodel 또는 SMC: CAD, HierarchicalStructures, HandoverDocumentation.

○ 세부 내용:

- 설계 변경 이력 추적: 장비 개조 시 변경된 MCAD 도면을 CAD 서브모델에 업데이트하여 현장 엔지니어가 항상 최신 도면을 기반으로 유지보수 작업을 수행하도록 지원합니다.

- 계층 구조 관리: HierarchicalStructures를 통해 장비를 구성하는 하위 모듈(펌프, 툴링 시스템 등)의 교체 이력을 BoM(Bill of Materials) 형태로 관리하여 자산 구성의 추적성을 확보합니다.

- 유지보수 문서 자동 매칭: 특정 부품 교체 시 해당 부품에 맞는 도면(CAD)과 교체 절차서(HandoverDocumentation)가 AAS 내에서 자동으로 연계되어 작업자에게 제공됩니다.

그림 47 장비 라이프사이클 및 구성 변경 관리

바. 기대효과

○ 분말성형공정 장비의 AAS 참조모델을 활용하면 제조사는 장비의 디지털 가치를 높이고, 운영사는 공정 운영의 투명성과 효율성을 극대화할 수 있습니다. 특히 MaterialManagement와 같은 특화 서브모델은 분말 제조 공정만의 전문성을 반영하여 고도화된 품질 관리를 가능케 합니다.